Architectural Processing and Inspections for Home Mortgage Insurance

Directive Number:  4145.1 REV-2

CHAPTER 1.  GENERAL INFORMATION	3

1-1.  INTENDED USERS	3

1-2.  FIELD OFFICE RESPONSIBILITY	3

1-3.  BUILDERS/DEVELOPERS AND HUD	3

CHAPTER 2.  ARCHITECTURAL EXHIBITS	3

2-1.   GENERAL	3

2-2.    DRAWINGS FOR INDIVIDUAL APPLICATIONS	4

2-3.  DESCRIPTION OF MATERIALS	9

2-4.   DRAWINGS FOR MASTER CONDITIONAL COMMITMENT (MCC)	10

2-5.  INDIVIDUAL WATER SUPPLY AND SEWAGE DISPOSAL SYSTEMS	12

CHAPTER 3 - PROCESSING PROCEDURES	14

3-1.   GENERAL	14

3-2.   SUBDIVISION PROPOSALS	14

3-3.   BUILDER CERTIFICATION PROCEDURE	15

3-4.   MANUFACTURED HOMES (MOBILE) FOR TITLE II MORTGAGE INSURANCE	25

3-5. PERMISSION TO START CONSTRUCTION (EARLY START)	31

3-6. CONDITIONS OF COMMITMENT	33

3-7. FEE ALLOCATIONS FOR ARCHITECTURAL REVIEWS	34

3-8.  DESK REVIEW OF STAFF AND FEE PERSONNEL	34

3-9.  RECONSIDERATION PROCEDURE	35

3-10. COMPLAINT PROCESSING PROCEDURES	36

3-11. PROPERTIES TO BE REHABILITATED (203k program)	36

3-12. SPECIAL MATERIALS, PRODUCTS AND METHODS OF CONSTRUCTION	36

3-13. PARTIALLY COMPLETED AND EXISTING CONSTRUCTION	36

3-14. CONDOMINIUM PROCESSING	37

3-15. REVIEW OF HUD ACQUIRED PROPERTIES	37

3-16. PROCESSING WATER AND SEWERAGE SYSTEMS	37

3-17. RECORDS DISPOSITION	37

3-18. BUILDER'S WARRANTY	37

3-19. AFFIRMATIVE FAIR HOUSING MARKETING PLAN	38

CHAPTER 4.  COMPLIANCE INSPECTIONS	38

4-1.   GENERAL	38

4-2.   INSPECTION RESPONSIBILITIES	38

4-3.  NUMBER OF INSPECTIONS	41

4-4.   CONSTRUCTION STATUS NOTIFICATION TO THE INSPECTOR	42

4-5.   PREMATURE CONSTRUCTION	43

4-6.   BUILDER REFUSAL TO UNCOVER CONCEALED CONSTRUCTION	44

4-7.   POSTING HUD CASE NUMBER	44

4-8.   POSTING EQUAL EMPLOYMENT OPPORTUNITY PLACARD (HUD 928.1)	44

4-9.   MANUFACTURED HOUSING INSPECTIONS	44

4-10.  DIRECT ENDORSEMENT INSPECTIONS	44

4-11.  REHABILITATION INSPECTIONS	44

4-12.  TRUSS FABRICATION INSPECTIONS	44

4-13.  REPAIR INSPECTIONS	45

4-14.  COMPLAINT AND STRUCTURAL DEFECT INSPECTIONS	45

4-15.  SUBDIVISION INSPECTIONS	45

4-16.  CASUALTY DAMAGE INSPECTIONS	45

4-17.  ROTATION OF INSPECTORS	45

4-18.  OFFICE REVIEW OF INSPECTION REPORTS (Form HUD 92051)	46

4-19.  ISSUANCE OF INSPECTION REPORTS	46

4-20.  UNREPORTED NON-COMPLIANCE	46

4-21.  EXTENSIVE NON-COMPLIANCES	46

4-22.  CONSTRUCTION CHANGES	46

4-23.  ACCEPTANCE OF CONSTRUCTION CHANGE(S)	47

4-24.  FIELD REVIEWS	48

4-25.  REVIEW OF MORTGAGEE CERTIFICATIONS	49

4-26.  INSPECTIONS BY LOCAL AUTHORITIES	49

4-27.  TRAINING of HUD Field Office	50

4-28.  HUD RECOMMENDATIONS FOR MODEL CODE CHANGE(S)	51

CHAPTER 5 - ESCROW PROCEDURES	52

5-1.   GENERAL	52

5-2.   MORTGAGEE'S ASSURANCE OF COMPLETION (Form HUD 92300)	52

5-3.    ESCROW AGREEMENT FOR POSTPONED OFF-SITE IMPROVEMENTS	57

5-4.    ASSURANCE OF COMPLETION BY A PUBLIC AUTHORITY	57

CHAPTER 6 - INSURED TEN-YEAR PROTECTION PLAN	57

6-1. GENERAL	57

6-2. ACCEPTANCE CRITERIA	60

6-3. MAXIMUM LOAN-TO-VALUE RATIOS FOR DWELLINGS WITH APPROVED	72

CHAPTER 7 - ARCHITECTURAL DATA	76

7-1.  GENERAL	76

7-2.  ARCHITECTURAL REFERENCE DATA	76

7-3. INSPECTION GUIDE	77

7-4.  MINIMUM PROPERTY STANDARDS (MPS)	78

APPENDIX 1 FILE IN PDF FORMAT	79

APPENDIX 2 BUILDER’S CERTIFICATION	79

APPENDIX 3 BUILDING CODE REVIEW	80

APPENDIX 4 POST-ENDORSEMENT/POST-COMMITMENT REVIEW	80

APPENDIX 5 AGREEMENT TO EXECUTE A BUILDER’S WARRANTY OF	80

APPENDIX 6 EARLY START LETTER	80

APPENDIX 7 ADDENDUM TO WARRANTY OF COMPLATION OF CONSTRUCTION	81

APPENDIX 8 SITE GRADING AND DRAINAGE GUIDELINES	81

APPENDIX 9 CONSTRUCTION EXHIBIT SUBMITTAL	85

APPENDIX 10 LIST OF HUD ACCEPTED	85

APPENDIX 11 ARCHITECTURAL REQUIREMENTS GRID	91

# CHAPTER 1.  GENERAL INFORMATION

1-1.  INTENDED USERS.  <TOP>This handbook is to be used by HUDJM

personnel, mortgagees, builders and developers who are

involved with proposed construction applications for one to

four-family properties and process cases through the HUD

Field Office.  Direct Endorsement Underwriters must also

apply these processing procedures.

1-2.  FIELD OFFICE RESPONSIBILITY.  <TOP>Mortgagees, builders and

developers must be informed by a HUD Field Office circular

letter of all modifications of a general nature to the

instructions and procedures in this handbook.

1-3.  BUILDERS/DEVELOPERS AND HUD.  <TOP>If a builder or developer wants to qualify a property for HUD mortgage insurance, a mortgagee

approved to do business with HUD must submit an application

for mortgage insurance for the property.  The builder or

developer may submit an application through a Direct

Endorsement (DE) Lender (Refer to Handbook 4000.4  for

instructions).

The application must be accompanied by all required

architectural exhibits (See Chapter 2).  Communication from

HUD concerning the property will be with the mortgagee.

HUD's issuance of a conditional commitment, or a Direct

Endorsement Lender's acceptance of an appraisal, is based on

a review of the architectural exhibits and HUD's inspections

during construction.  This neither precludes a review or

inspection required by a locality nor relieves a builder of a

contractual obligation to a homeowner.

Architectural Processing and Inspections for Home Mortgage Insurance

Directive Number: 4145.1

4145.1 REV-2

# CHAPTER 2.  ARCHITECTURAL EXHIBITS

2-1.   GENERAL.  <TOP>Construction exhibits will be submitted with

each application for mortgage insurance that involves

proposed construction, partially completed construction

or additions (alterations) to existing construction.

Exhibits for alterations or repairs need only pertain

to the work to be done.

Exhibits must be adequate and accurate to determine

compliance with applicable HUD standards, form the

accurate basis for HUD commitments, determine

acceptability of the physical improvements, and provide

the basis for conclusions involving the builder's

warranty.

Exhibits must be submitted for all properties receiving

high loan-to-value ratios.  All exhibits must be

retained in the HUD case binder in order to discuss

construction complaints with homeowners or builders.

Homeowners may request copies of exhibits under the

Freedom of Information Act.

Refer to the Architectural Requirements Grid in

Appendix 11.

2-2.    DRAWINGS FOR INDIVIDUAL APPLICATIONS. <TOP>Submit with each

application:

A.  Plot Plan.  Minimum scale of 1" = 20'- 0" or

1/16" = 1'- 0", showing (See example of Typical

Plot Plan in Appendix 1):

1)   Lot and block number;

2)   Dimensioned length of each boundary;

3)   North point;

4)   Dimensions of front, back, and side yards;

5)   Location and dimensions of garage, carport, and

other accessory buildings, including footing

drains (or other subdrains) and their points

of discharge;

2-1                   3/90

4145.1 REV-2

(2-2.A.)

6)  Location of streets, curbs, walks, driveways,

approach slabs, surfacing and utilities;

7)  Location of steps, terraces, porches, fences,

trees, shrubs, retaining walls, slopes, and

drainage swales, channels, pipes and related

facilities;

8)  Location and dimensions of easements and

established setback requirements, if any.

9)  Grade elevations will be provided at:

a.  First floor of dwelling and floor of

garage, carport and other accessory

buildings;

b.  Finish curb or crown of street at points of

extension of lot lines;

c.  Existing and finish grade elevations at

each corner of the plot and each principal

corner of the dwelling;

d.  Finish grade elevations at the toe of any

slopes or retaining walls;

e.  Other site elevations necessary to show

proper grading design and the flow routes

of surface drainage, including but not

limited to: heads of swales; points of

change in swale gradients; at all building

walls; and on proposed flatwork that might

trap runoff (See Appendix 8);

f.  Detailed existing and finish grade

elevations are required where topography,

or design of the structure, necessitates

special grading, drainage, subdrainage,

slope stabilization or foundation design;

Examples are:  irregular or steeply sloping

sites; filled or cut areas or multi-level

structures; and

3/90                       2-2

4145.1 REV-2

(2-2.A.9))

g.    Where fill depths will exceed two feet

beneath dwellings, or where expansive,

compressible, collapsing or organic soils

will be present beneath buildings, grading

plans and specifications developed

according to procedures in HUD Handbook

4140.3  (Data Sheet 79G), must be a

commitment requirement and complied with

during construction;

This requires pre-design site

exploration and testing and post-grading

engineering compliance reports.

B.   Floor Plan.  Minimum scale of 1/4" = 1'- 0",

showing:

1)    Provide separate foundation plan with

construction details, including any subdrainage

facilities.  Where on-site soils are expansive,

compressible, collapsible or organic, or where

subsidence is possible, foundation plans must

be accompanied by supporting soil, geologic,

groundwater and structural design information.

In outlying low volume areas where

qualified engineering services may be

locally unavailable, or the incidence and

significance of observable foundation

performance problems in and around the

location of the subdivision and property

are negligible, HUD Field Office can

determine the degree of refinement needed

for the supporting engineering data.

2)    Plan of each floor and of basement, if any.

If dwelling is of crawl space type, or

slab-on-grade, provide separate foundation plans

showing construction details.

3)    Plan of any attached terrace, porch, garage, or

carport.

2-3                      3/90

4145.1 REV-2

(2-2.B.)

4)  Direction, size and spacing of all floor and

ceiling framing members, girders, columns or

piers.

5)  Location of all partitions and walls,

indicating door and window sizes and direction

of door swing.

6)  Location and size of all permanently installed

construction and equipment (i.e., kitchen

cabinets, closets, shelves, plumbing fixtures,

water heaters & heating-cooling units).

Details of kitchen cabinets may be on separate

drawings.

7)  Location and symbols of all electrical

equipment, including switches, outlets,

fixtures and panels.

8)  Heating system on separate drawings, or as part

of plan of floors and basement, showing:

a.  Location and size of ducts, piping,

registers, radiators, etc;

b.  Location of heating unit and room

thermostat;

c.  Total calculated heat loss of dwelling.

For duct or piped distribution system

include calculated heat loss of each heated

space, using American Society of Heating,

Refrigerating and Air Conditioning (ASHRAE)

Standards; and

d.  Model number and BTU/hour capacity of

equipment.

3/90                             2-4

4145.1 REV-2

(2-2.B.)

9)  Cooling system, on separate drawings or as part

of heating plan, floor or basement plan,

showing:

a.  Location and size of ducts, registers,

compressors, coils, etc;

b.  Heat gain calculations, including estimated

heat gain for each space conditioned;

c.  Model number and BTU/hour capacity of

equipment or units according to applicable

ASHRAE standards;

d.  BTU/hour capacity and total KW input at

stated local design conditions; and

e.  If room or zone conditioners are used, show

location, size, and installation details.

C.  Exterior Elevations.  Minimum scale of 1/4" =

1'- 0", must show:

(Elevations other than main elevation, which

contain no special details, may be drawn at

1/8" = 1' - 0".)

1)  Front, rear, and both side elevations, and

elevation of any interior courts;

2)  Window and doors - show size unless separately

scheduled or shown on floor plan;

3)  Wall finish materials - show where more than

one type is used;

4)  Depth of wall footing, foundations, or piers;

show stepped, if at more than one level;

5)  Finish floor lines; and

6)  Finish grade lines at buildings.

2-5                            3/90

4145.1 REV-2

(2-2.)

D   Sections.  Minimum scale of 3/8" = 1'- 0", will be

provided through the following areas of the

structure:

1)  Exterior wall sections must show details of

construction from the bottom of the foundation

to the highest point of the roof;

Where more than one type of wall

construction is used, each type must be

shown.

2)   Show wall sections through any portion of the

dwelling where rooms are situated at various

levels or where finished attic space is

proposed; and

3)   Stairwells, landings, and stairs, including

headroom clearances and surrounding framing.

E.  Details.  Provide the following:

1)   Elevations and sections through fireplace,

scale not less than 3/8" = 1'- 0";

2)   Engineered design with roof truss connections

and test data, scale not less than 3/8" =

1'- 0";

3    Elevations and sections through kitchen

cabinets, indicating shelving, scale not less

than 1/4" = 1'- 0"; and

4)   Sections and details of all critical

construction points, special structural items

or special millwork, scale not less than 3/8" =

1'- 0".

F.  Additional Exhibits.  Field Office can require

technical reports and/or other exhibits when the

mortgage risk could be affected by unstable soil or

other differential ground movement, ground water

3/90                             2-6

4145.1 REV-2

(2-2.F.)

problem, and other site or toxic hazards.  Refer to

paragraph 7-4.

Examples include, but are not limited to:

engineers' reports on soil exploration and testing;

earthwork specifications (79-G) and special grading

plans; special foundation and related designs

proper for conditions found; slope or other

stability evaluations, evaluations of underground

sewage, effluent disposal and waste disposal sites.

G.   Size of Construction Drawings.  On individual new

construction cases plans should be no larger than

18" X 24".  Half size photocopies of the drawings

(8 1/2" X 14") are acceptable provided they are

legible and lettering is no smaller than 1/16".

Computer aided drawings at half scale are also

acceptable.  See Appendix 9.

2-3.  DESCRIPTION OF MATERIALS. <TOP>

A.   Submit Form HUD 92005 for each plan type.

Instructions are on it.

B.   Builder May Provide Own Form (computerized is

acceptable).  Format and printed text must be

identical to Form HUD 92005.

2-4.   DRAWINGS FOR MASTER CONDITIONAL COMMITMENT (MCC) <TOP>.

Submit only for group applications that repeat a basic

type dwelling (see list of additional items in Handbook

4115.3).

A.  Master Plot Plan will include:

1)  Scale that will clearly and legibly show

information;

2)  North point;

3)  Location and width of streets and rights-of-way;

2-7                          3/90

4145.1 REV-2

(2-4.A.)

4)  Locations and dimensions of all easements;

5)  Boundary dimensions of each lot;

6)  Dimensions locating each dwelling on the lot;

7)  Dimensions of front, back, and side yards;

8)  Location and dimensions of garages, carports,

or other accessory buildings;

9)  Location of walks, driveways, and other

permanent improvements; and

10) Identification of each lot by number and

indication of basic plan and elevation.

B.   Typical Plot Plan.  Submit for each basic, type

dwelling in lieu of fully detailing each lot on

Master Plot Plan (See paragraph 2-2.A. and

Appendix 1).  Use only when topography and lot

arrangements present no collective (or individual

planning) construction problems.  Necessary

information not shown on typical plot plan will be

included on Master Plot Plan.  See Appendix 8 for

guidance on grading and drainage.

A plot plan for each individual lot must be

provided to the fee inspector at the initial

inspection on site.

C.  Grading and Drainage.  Show on a separate plan or

on the Master Plot Plan.  Refer to HUD Handbooks

4140.1 and 4140.3. See Appendix 8.

1)  Scale must show contours of existing and finish

grades at intervals of not more than 5 feet;

2)  Contour intervals less than 5 feet may be

required for less steeply sloping lots;

3)  Location of house and accessory buildings on

each lot;

3/90                            2-8

4145.1 REV-2

4145.1 REV-2

(2-4.C.)

4)  Identification of each lot by number;

5)  Elevations according to individual plot plan,

including bench mark and datum or, in lieu of

finish grade elevations, contours of proposed

finish grading may be submitted.  Contour

intervals selected will be appropriate to the

topography of the site;

6)  Lot grading will be shown by indicating

protective slopes and approximate location of

drainage swales; and

7)  Location of drainage outfall if any drainage is

not to a street.

Special requirements by HUD Field Office may be

necessary for grading plans, specifications and

engineers certifications (79-G).  See paragraph

2-2.B. and 2-2.F.

D.  Floor Plans, Elevations, Sections, and Details.

Submit for each plan type (see paragraph 2-2 for

scale and details).

Optional elevations to a basic plan can be shown at

a scale not less than 1/8" = 1'- 0".

When the Schedule of Options is used, provide

necessary exhibits.

E.  Description of Materials.  See paragraph 2-3.

2-5.  INDIVIDUAL WATER SUPPLY AND SEWAGE DISPOSAL SYSTEMS<TOP>.

Provide:

A.  A Written Opinion by the Health Authority.

1)  Water-supply system.

Give the most recent record of failure of wells

in the immediate vicinity to furnish an

adequate supply of water.

2-9                     3/90

4145.1 REV-2

CHANGE 1

(2-5.A.)

2)  Sewage disposal system.

a.  Provide a statement that the permitted

individual sewage disposal system, with proper

maintenance, can be expected to function

satisfactorily and is unlikely to create an

unsanitary condition;

The statement does not constitute an assurance

by the authority that the system will continue

to perform satisfactorily, only that if properly

maintained, the system can be expected to be

adequate, based on the authority's knowledge of

present site conditions; and

b.  Show how many bedrooms the sewage disposal

system is designed for and whether there is or

is not a garbage disposal.

The HUD Field Office or the DE Lender must include a

Specific Condition on Form HUD 92800.5B (in addition to

item E on the back of the form that ensures acceptable

installation according to accepted exhibits).  Obtain a

statement from the local health authority that if a

water-supply system is used, that the quality of water

is satisfactory for human consumption.

B.  Location Map of property.

C.  Plot Plan.  Show these additional items (see paragraph

2-2.A. and Appendix 1):

1)  Lot lines and all improvements;

2)  Topography;

3)  Location of septic tank, distribution box,

absorption field or bed, seepage pits, and other

essential parts of sewage system;

Holding tanks are not acceptable on newly

constructed properties.

2/92                        2-10

4145.1 REV-2

CHANGE 1

(2-5.C.)

4)  Location of well, service lines and other essential

parts of water supply system, including well casing

seal;

5)  Distance to individual well from the dwelling,

septic field and property line.  Also show distance

of well to individual well and septic systems on

adjacent properties;

6)  Grade elevations of the well and/or septic systems;

and

7)  Location of the individual systems on adjacent

properties and distance to poisoned soil.

If wells are not involved on the subject property, or

adjacent properties, only that part of the sewage system

on adjacent property within 10 feet of the property line

need be shown.

D.  Report on Local Conditions.

1)  Hydrology and any protection required;

2)  Geology, including a description of soil materials

to a depth of at least six feet; and

3)  Topography.

E.  Plans and Specifications of systems.  Show details of

all component parts; indicate material, equipment and

construction.

F.  Well Log and Yield Report.

G.  Water Quality Report on bacteriological properties.   *

2/92

2-11

(2-4.C.)

Architectural Processing and Inspections for Home Mortgage Insurance

Directive Number: 4145.1

4145.1 REV-2

# CHAPTER 3 - PROCESSING PROCEDURES

3-1.   GENERAL. <TOP>Architectural analysis is based on an evaluation

of the physical characteristics and proposed improvements to

a property to determine whether they meet HUD's requirements

for mortgage insurance eligibility.  Submittal of the

Architectural Requirements is described in Appendix 11.

It is assumed that "proposed construction" cases,

"under-construction" cases and "existing construction" cases

(additions) will be completed according to the accepted

exhibits, local codes and ordinances, HUD requirements and

applicable commitment conditions.

"Physical Improvements" refers to the entire property,

including land, building(s) and appurtenances comprised by

the mortgage security.  Properties are analyzed apart from

their neighbors, except for known conditions on adjoining

lots that may affect health and safety of the occupants.

Health and safety items include, but are not limited to:

inadequate surface drainage; potential for high ground

water levels; earth faults; landslides; unstable soils;

or toxic wastes.

3-2.   SUBDIVISION PROPOSALS <TOP>.  Process according to:

A.  Handbook 4135.1 , Procedures for Approval of Single

Family Proposed Construction Applications in New

Subdivisions.

B.  Handbook 4115.3, Master Conditional Commitment

Procedure.

3-3.   BUILDER CERTIFICATION PROCEDURE <TOP>.

A.  Applications for Conditional Commitment and VA-CRV

(MCRV)  Proposed Construction Cases - must comply with

regulations in 24 CFR 200.926 and construction exhibit

requirements in Chapter 2 of this handbook.

B.  HUD Requirements in the regulation are to be used for

all individual and group (Master Conditional Commitment)

applications for high loan-to-value ratios.  For

applications on dwellings "under-construction" or

"existing less than one-year," with a HUD accepted

3-1                        3/90

4145.1 REV-2

(3-3.B.)

insured ten-year protection plan, see paragraph 6-3.

C.   HUD Field Offices will accept certification (Form HUD

92541; see Appendix 2) by (1) the builder, and (2) a

builder or the builder's architect or other qualified

agent, for compliance with local or State building codes

that were accepted by the HUD Field Office according to

24 CFR 200.926a or National Building Code criteria.

The builder must certify on bottom portion of form

on all cases.  A builders agent may only certify on

the top portion of the form.

This certification process is mandatory for all

properties receiving high loan to-value financing or on

properties not yet completed at the time, the appraisal

is requested.  If future field reviews or homeowner

complaints reveal a pattern of non-compliances, use

procedure described in paragraph 3-3.K.2)f.

The  certification process will also be used for obtaining

high loan-to-value ratios for dwellings covered by HUD

accepted warranties (after construction has begun or the

dwelling is existing-less than one-year old).  Instructions

are in paragraph 6-3 of this handbook.

D.   Mortgagee Responsibilities.

1)   Obtain current list of HUD accepted building codes.

2)   Ensure builder has attached the proper Builder's

Certification (Form HUD 92541; see Appendix 2)  and

the Agreement to Executed a Builder's Warranty of

Completion of Construction (Form HUD 92541-A) on the

Front page of each set of plans prior to submitting

application for conditional commitment.

a.   It is not necessary for mortgagees to review the

plans; however, lender should ensure that the

required construction exhibits are present.

Ensure builder is on the list of certified

builders (See paragraph 3-3.K.3)).  If

builder is not on list, exhibits must be

reviewed by HUD Field Office.

3/90                              3-2

4145.1 REV-2

4145.1 REV-2

(3-3.D.2)

b.  On individual proposed VA-CRV's, certifications

must be attached to each case.  On VA Master-CRV'S,

certifications for each model type are

required; copies of the certifications are

placed in each file when converted to HUD.

Plans need not accompany VA-CRV's.

Where applicable, submit the Warranty of

Completion of Construction (Form HUD 92544) and

the Addendum to the Warranty of Completion of

Construction (Form HUD 92544-A);

3)  Call HUD assignment desk for case number and fee

appraiser's name;

4)  Send the following to the fee appraiser:

a.  Two sets of construction exhibits (with case

number on the plans and certification attached).

If builder wants a copy, submit three sets; and

b.  Form HUD 92800, Application for Property

Appraisal and Commitment;

5)  If the Master Conditional Commitment procedure is

used, see Handbook 4115.3 for instructions; and

6)  Upon receipt of the "Inspection Copy" of the

construction exhibits (after issuance of the

Conditional Commitment/DE Statement of Appraised

Value) send it to the fee inspector.

Inspection Copy may be forwarded to builder for

availability to the fee inspector at time of

inspection.  If 3 sets of exhibits were

submitted in 4)a. above, "Builder's Copy" should

be sent to builder.

E.  Builder Responsibility.  Assure that construction

exhibits comply with:

1)  Local, State, or model code requirements (see

criteria in 24 CFR 200.926, Minimum Property

Standards for One- and Two-Family Dwellings);

3-3              3/90

4145.1 REV-2

CHANGE 1

(3-3.E.)

2) HUD requirements contained in 24 CFR 200.926d; and

3) Chapter 2 of this handbook (Architectural Exhibits).

An original signed copy of the certification by the

builder and/or the builder's agent (Form HUD 92541; see

Appendix 2)  is required by HUD and must be completed

and attached to the cover sheet of the plans.  Also

complete Form HUD 92541-A, Agreement to Execute a

Builder's Warranty of Completion of Construction (Form

HUD 92541).

F.  Fee Appraiser Responsibilities.

1)  Assure that the construction exhibits in chapter 2

are sufficient to make an appraisal.

Do not make the appraisal if the proper builders

certification is not with each set of plans.

Return to mortgagee.

2)  After completing the appraisal, forward 92800

package and two sets of construction exhibits to the

HUD Field Office.

For Direct Endorsement cases, return the

appraisal package, including two sets of the

construction exhibits, to the mortgagee.  Send a

copy of the appraisal to HUD.

G.  Assignment Clerk Responsibilities.

1)  Receive call from mortgagee requesting assignment of

case number and name of appraiser.

a.  If subdivision is not approved, no case number

is to be assigned.

*    b.  If subdivision is approved continue processing.  *

2/92                          3-4

4145.1 REV-2

(3-3.G.)

2)  Enter information about the mortgagee and the

property on CHUMS Receiving/Assignment Screen.

CHUMS verifies that the mortgagee is eligible,

assigns next available case number and a fee

appraiser, unless assignment is delayed.

CHUMS assigns the appraiser with the lowest

caseload who can take assignments in that

geographic area.  Override the system, if

appropriate.

3)  Check the assigned number off the Available Case

number listing.

4)  Prepare and file case binder in pending file.

H.   Receiving Clerk Responsibilities.

1)  Upon receipt of the appraisal package:

a.  Review the appraisal for completeness, ensure

that all plans have the correct case number and

the builder's certification (original signed

copy) placed on them.

b.  Date stamp appraisal and place all documents in

appropriate case binder.  Enter date received on

CHUMS Appraisal Receiving Logging Screen.

1.  One set of construction exhibits on left

side of the case binder.

2.  Form HUD 92800 (right side of case binder).

3.  One set of construction exhibits will be

stamped "Inspection Copy," placed loose in

the binder.  If third set is submitted,

stamp it "Builder's Copy."

2)  Forward case binder to:

a.  The Valuation Branch if the builder is on the

certified list (obtain list from A & E).

b.  Architectural Branch if builder is not on

certified list.

3-5                       3/90

4145.1 REV-2

(3-3.)

I.   Review Appraiser Responsibilities.  For each application

requesting high loan-to-value ratio on a proposed or

newly constructed property:

1)  Check to see that Form HUD 92541, Builder's

Certification (original signed copy) and Form HUD

92541-A, Agreement to Execute a Builder's Warranty

of Completion of Construction (original signed copy)

is attached to the plans, properly completed and

that the correct building code(s) was used.

2)  Make a cursory review to determine if required

exhibits have been submitted (see chapter 2).

Missing exhibits (minor) should be listed as

conditional commitment requirements;

If construction exhibits are insufficient,

suspend processing (use Form HUD 92026) or refer

it to the Architectural Branch according to

paragraph 3-3.K.2)f.

3)  Review appraisal according to Handbook 4150.1.  When

completed, date and sign (include CHUMS ID number on

appraisal report); and

4)  Forward case binder to Commitment Clerk.

J.   Commitment Clerk Responsibilities.

1)  Review the case binder for any specific commitment

conditions set forth by the Valuation Branch or the

Architectural Branch;

2)  Enter information on CHUMS Appraisal Disposition

Screen about property, any commitment conditions,

assignment of fee inspector (include address and

telephone number) or, if applicable, reasons

for rejection;

3)  Print a Conditional Commitment (Form HUD 92800.5B)

or prepare a Report on Application (Form HUD

92026);

3/90                           3-6

4145.1 REV-2

(3-3.J.)

4)   Send to the mortgagee:

a.  Conditional commitment;

b.  Inspection copy of construction exhibits; and

c.  Builder's copy if third set was submitted in

paragraph 3-3.4)a.

5)   Daily, provide Architectural Branch with case

numbers and the builder's name from all commitments

issued on proposed construction cases (include names

of builders who requested high loan-to-value

financing with a HUD accepted insured ten-year

protection plan; and

6    Forward case binder to files.

K.  Architectural Staff Responsibilities.

1)   Obtain copies of State and local codes and compare

table of contents with criteria in 24 CFR 200.926a

(see Appendix 3).

Maintain a list of acceptable, partially acceptable,

and unacceptable codes by locality.  Give to

Valuation Branch, approved mortgagees and, upon

request, to builders.  Provide up-to-date list to

Regional Office semi-annually.

2)   Completely review the first submission of

construction exhibits, for each builder, for

compliance with the HUD standards in 24 CFR

200.926d "Construction Requirements" (not the

accepted local, state or national building code).

See Appendix 8.

a.  Ensure all construction exhibits (see paragraph

2-2) are present and that the exhibits are

sufficient to make an inspection of the

property.

Fee inspectors should review plans for

compliance with applicable building codes.

3-7                     3/90

4145.1 REV-2

(3-3.K.2))

b.  Review Form HUD 92541, Builder's Certification,

and Form HUD 92541-A, Agreement to Execute a

Builder's Warranty of Completion of Construction

to assure that the applicable building code(s)

and HUD construction requirements were properly

listed.

c.  Amend exhibits, wherever possible, show minor

non-compliances (or omissions) with HUD

standards to avoid rejection of the case.

Notify builders of any non-compliances.

1.   Amendments made during processing will be in

color "red" with the initial "HUD" for ease

in identifying the changes.

2.   When an Amendment Sheet is used, the

non-compliance items are keyed to the items

on the sheet.

3.   A special condition of the commitment (see

paragraph 3-6.C.) can be made if amending of

exhibits is not feasible.  Enter on

Conditional Commitment or reference on form

and attach conditions to it.

4.   When contradictory or alternate items

(materials, options, etc.)  are included in

the construction exhibits, the one

customarily used in the industry is retained

and the other deleted.

d.  Design recommendations for reasonable and

practical improvements can be provided as rough

sketches on tracing paper or on the exhibits.

1.   Clearly distinguish as recommendations, and

not amendments.

2.   Record comments on all exhibits.

e.  If a builder's submission is acceptable,

post-review future applications from the builder

(see 4) below).

3/90                             3-8

4145.1 REV-2

(3-3.K.2))

f.  If a builder's submission is unacceptable, note

deficiencies on Form HUD 92026, Report on

Application.

1.  The Director of Housing can require that a

licensed architect or engineer certify

future submissions and prohibit the builder

from certifying the plans, if continuing

reviews, or homeowner complaints, show that

builder certified erroneously on major

items .

2.  If problems continue, use Limited Denial of

Participation (LDP) procedures in 24 CFR,

Part 24, Subpart D (note 24.6(a)(6)).

3)  Maintain a current list of builders that have

acceptably certified.  Distribute to the Valuation

Branch, receiving clerk, to mortgagees and, upon

request, to builders.  Builders that have converted

cases from VA-CRV's (or MCRV'S) or are requesting

high loan-to-value ratios with a HUD accepted

ten-year protection plan should not be placed on the

list (See paragraph 3-3.D.2)b.).

List should provide the complete name under

which the builder is certified and the names of

the individuals authorized to sign for the

builder .

Builders who are rejected on their first submission

of a case should not be placed on the list upon

resubmission of that case.  To be placed on the

list, submission should be correct initially.

Do not place a builder on the list if it is

known that he/she will not build more than one

house per year.  Homeowners acting as the

builder should not be placed on the list.

Review the builders list yearly.  If a builder has

not done business with HUD within a 12 month period,

remove them from the list.

3-9                      3/90

4145.1 REV-2

CHANGE 1

(3-3.K.)

4)  Post-Commitment Reviews (Form HUD 92542; see

Appendix 4) can be initiated by Chief Architect if a

builder is not properly certifying.  Review can only

determine compliance with the construction

requirements of 200.926d.

a. Notify mortgagee and builder in writing of any

non-compliances.  Send copy to fee inspector.

b. Place copy in builder's file.  Documentation of

the file is necessary if the Director of Housing

requires a registered architect or engineer to

certify the plans or a LDP is issued.

5)  Post-Endorsement Reviews (Form HUD 92542; see

Appendix 4) are completed on Direct Endorsement

Lenders according to instructions in HUD Handbook

4000.4.  The review can only determine compliance

with construction requirements in 200.926d.

*   If the review finds the builder to have acceptably

certified, note on the Underwriting Report (Form HUD

54118) "Builder Certification rated Good, Form HUD

92542 not required."  If builder needs comment or

guidance, note on Underwriting Report, "See attached

Form HUD 92542;" maintain a copy in builder's file

and send a copy directly to builder.                  *

L.  Master Conditional Commitment (MCC).  This procedure may

be used by all builders who want HUD insured financing

available for a group of five or more lots within a

subdivision.  See HUD Handbook 4115.3 for additional

instructions.  Builders Certification (Form HUD 92541)

and the Agreement to Execute a Builder's Warranty of

Completion of Construction (Form HUD 92541-A) must be

attached to front cover of each set of construction

exhibits.  Copy of certification must be included with

closing documents for each case.

1)  Builders who do not use the MCC process will submit

an application for each individual case (with all

exhibits).

2)  After MCC is issued, Valuation Branch delivers

construction exhibits to the Architectural Branch

for review and for establishing the "basic case"

file (use Form HUD 92014d) for each model type.

2/92                          3-10

4145.1 REV-2

(3-3.L.2)

a.  On a post-commitment basis, A & E reviews each

"basic model" for compliance with exhibit

requirements in chapter 2 and HUD standards in

24 CFR 200.926d (Also see Appendix 8).  Master

plot plan and subdivision grading/drainage plan

are also reviewed.

1.  Notify mortgagee and builder in writing of

any non-compliances.

2.  Send copy to fee inspector.

b.  Field review of inspections are required in

paragraph 4-24.

3)  Direct Endorsement Lenders may use a Master

Appraisal Report procedure.  Refer to HUD Handbook

4115.3 for instructions.

M.  Direct Endorsement Lender.  Refer to Handbook 4000.4.

Once a builder's first submission has been accepted by

HUD, it will not be necessary for the lender to review

plans and specifications for compliance with 24 CFR

200.926d.  Obtain the current list of acceptable

building codes from HUD Field Office.

1)  Ensure that proper construction exhibits have been

submitted by builder (Chapter 2).  The Builder's

Certification (Form HUD 92541) and the Agreement to

Execute a Builder's Warranty of Completion of

Construction (Form HUD 92541-A) are attached to the

front page of the construction exhibits.  Ensure

that the wording on the certifications are correct.

2)  Post-Endorsement reviews will be performed by HUD.

3)  Submit and certify:

a.  Date of agreement to provide HUD insured

financing, and

b.  Date on which on-site construction began.

3-11                       3/90

4145.1 REV-2

3-4.   MANUFACTURED HOMES (MOBILE) FOR TITLE II MORTGAGE INSURANCE <TOP>.

A.  General Eligibility Criteria (Refer to 24 CFR 203.43f).

1)  The home must have a floor area of no less than 400

square Feet;

2)  The home must be constructed in conformance with the

Federal Manufactured Home Construction and Safety

Standards, as evidenced by an affixed certification

label, according to 24 CFR 3280.8;

Only manufactured homes produced after June 15,

1976, will bear that seal.  Manufactured homes

produced prior to that date are ineligible for

insured financing under Title II.

3)  The home must be classified and taxed as real

estate;

4   The mortgage must cover both the manufactured unit

and its site and have a term of no more than 30

years from the date amortization begins; and

5)  The manufactured unit must not have been installed

or occupied previously at any other site or

location.

6)  The finished grade elevation beneath the

manufactured home or, if a basement is used, the

lowest finished exterior grade adjacent to the

perimeter enclosure, must be at or above the

100-year return frequency flood elevation.

This requirement applies wherever manufactured homes

may be installed, not just in locations designated

by the National Flood Insurance Program as areas of

special flood hazard.

3/90                           3-12

4145.1 REV-2

(3-4.)

B.  Criteria for Proposed Construction Properties.  Refer to

the Architectural Requirements Grid in Appendix 11.  In

addition to the general eligibility criteria;

1)   They must have, with or without a basement, a

site-built permanent foundation that meets or exceeds

applicable requirements of 24 CFR 200.926.  Comply

with Handbook 4930.3, Permanent Foundations Guide

for Manufactured Housing;

For  an "Existing" property, use HUD Handbook

4930.3, Appendixes A, B and C to verify the

design of the existing system.  See paragraph

3-4.C. for additional information.

2)   They must he permanently attached to that foundation

by anchoring devices adequate to resist all loads

identified in 24 CFR 200.926d (this includes

resistance to ground movements, seismic shaking,

potential shearing, overturning and uplift loads

caused by wind, earthquake, etc.);

Anchoring straps or cables affixed to ground

anchors, other than footings (or piers), will not

meet this requirement.  The unit must be anchored

to the footing (or pier);

3)   They must have permanent utilities, installed and

protected from freezing;

4)   The towing hitch or running gear must be removed

(Including tongues, axles, brakes, wheels, and

lights).  The chassis must stay in place:

A chassis that has been removed from a

manufactured unit is unacceptable for Title II

programs;

5)   There must be a properly enclosed crawl space with a

continuous permanent foundation-type construction

(similar to a conventionally built foundation, i.e.,

concrete, masonry or treated wood).  The perimeter

3-13                         3/90

4145.1 REV-2

(3-4.B.5))

enclosure, if separate from supporting the

foundation, must:

a.   Be designed to resist all forces to which it may

be subject without transmitting to the building

superstructure any movements or effects caused by

frost heave, soil settlement (consolidation), or

the shrinking or swelling of expansive soils;

b.   Be adequately secured to the perimeter of the

unit to exclude entry of vermin and water; and

c.   Allow proper ventilation of the crawl space.

6)  The site, site improvements, and all other features

of the property (exclusive of the manufactured

living unit) not addressed by the Federal

Manufactured Home Construction and Safety Standards,

must meet or exceed applicable requirements of 24

CFR 200.926d (except 200.926d(c)(4)(i)); See

Appendix 8;

7)  The manufactured unit must be braced and stiffened

before it leaves the factory to eliminate racking

and potential damage during transportation;

8)  It must be eligible for high-ratio insured financing

according to 24 CFR 203.18(a)(2).  Acceptance for

mortgage insurance is satisfied by interpretation of

the beginning of construction as the "commencement

of onsite construction."  The manufactured home must

not have been installed or occupied previously at

any other site or location.

9)  The manufactured unit must be insulated so that the

envelope "Uo" value (calculated according to NFPA

501, BM-1976) does not exceed:

a.   0.145 in Climatic Zone I, includes Alabama,

Arizona, Arkansas, California, Florida, Georgia,

Hawaii, Louisiana, Mississippi, New Mexico,

North Carolina, Oklahoma, South Carolina,

Tennessee and Texas;

3/90                             3-14

4145.1 REV-2

CHANGE 1

(3-4.B.9)

b.  0.087 in Climatic zone III, includes Alaska,

Maine, Michigan, Minnesota, Montana, New

Hampshire, North Dakota, South Dakota, Vermont,

Wisconsin and Wyoming; and

c.  0.099 in Climatic Zone II, the remainder of

states;

*       Prior to the manufactured unit being delivered to

the site and/or placed on the foundation, a

Manufacturer's Certificate must be provided to the

lender showing the following information:

-   FHA Case Number and address of the property;

-   The label (metal plate) number of the unit as

shown on the Federal Manufactured Home

Construction and Safety Standard Data Plate on

the exterior of each home;

-   The Climatic Zone (I, II or III) the unit was

designed for, according to NFPA 501 BM-1976; and

-   The overall coefficient of heat transmission

("Uo" value) calculated in accordance with NFPA

501 BM-1976.

10) The fee inspector must verify the Uo value on each

units Data Plate.

a.  Heat loss requirements for Zone I will be

fulfilled in any manufactured home bearing a

Data Plate indicating compliance with MHCSS Zone

II (Data Plate labeling for Zone III is

acceptable for Zone II).  Where this is the

case, heat loss calculations for the unit need

not be submitted.

b.  Whenever Uo value calculations are certified by

a manufacturer's Design Approval Primary

Inspection Agency (DAPIA) as being equal to or

more stringent than this criteria, it may be

assumed that the manufactured home complies with

these heat loss requirements.

3-15                   2/92

4145.1 REV-2

CHANGE 1

(3-4.B.)

11) If the field office has a question as to the

acceptability of the unit's construction, the unit

should be determined unacceptable pending a review

by the applicable State Administrative Agency or HUD

Headquarters Staff (Manufactured Housing Division).

12) There must be a Builders Certification, Form HUD

92541 (Appendix 2) for all site work, including the

permanent foundation (see paragraph 3-3).

a.  Do not review the construction exhibits for the

dwelling superstructure for compliance with the

Federal Manufactured Home Construction and

Safety Standards.

Construction exhibits for the unit consist

of a description of materials and a floor

plan.  Use for appraisal purposes only.

b.  Exhibits to submit for review:

1.  Builders certification for manufactured

housing (Form HUD 92541; see Appendix 2);

2.  Agreement to Execute a Builder's Warranty of

Completion of Construction (Form HUD 92541-A

see Appendix 5);

3.  Floor plan of the unit (this may be a

brochure type floor plan that shows the

layout of the home, with no scale;

4.  Description of Materials;

Describe the type of materials used to build

the unit.  It does not have to be elaborate

and can merely state the type of siding,

roofing, carpeting, etc.  For site and

foundation work, use Form HUD 92005.

5.  Design calculations, details and drawings

for installation, anchorage and construction

2/92                          3-16

4145.1 REV-2

CHANGE 1

(3-4.B.12))           details of permanent foundation and

perimeter enclosure;

6.  Heat loss calculations from manufacturer to

show compliance with paragraph 3-4.B.9) and

10).  Submit insulation details when

calculations are required;

7.  Site elevation determinations related to

potential flooding must be based upon

information shown on National Flood

Insurance Rate Maps, where available;

In all other cases, use Regional Civil

Engineering staff for determinations.

8.  Plot plan (see paragraph 2-2.A.);

9.  Additional exhibits, where necessary, to

ensure site acceptability (see paragraph

2-2.F.); and

10. Individual water supply and sewage disposal

systems (see paragraph 2-5).

c.  The intermediate (framing) inspection customary

for site-built housing will not be required.

C.  Criteria for Existing Properties.  Refer to Appendix 11,

Architectural Requirements Grid.  Refer to HUD Handbook

4150.1 for appraisal criteria.

The foundation design information in HUD Handbook

4930.3, Permanent Foundations Guide for Manufactured

Housing, Appendices A, B and C, may be used to verify

the design of the existing system.  Provide a structural

engineers certification to verify compliance with the

handbook guidelines and with the requirements set forth

in paragraph 3-4.B.

Existing manufactured homes must not have been installed

or occupied on any other site and must comply with items

B-1) through item B.6) and item B.9) as listed for

proposed construction.  Mortgage insurance is not

allowed if the manufactured home was moved from another

3-17              2/92

4145.1 REV-2

CHANGE 1

(3-4.C.)

site.  Also comply with the general eligibility criteria

in paragraph 3-4.A.

Manufactured units which have been erected on a site for

one year or less prior to the date of application for

mortgage insurance and which were not approved by HUD or

a DE Lender prior to their construction on site, are

subject to a loan-to-value limitation of 90 percent (for

a principle resident) of the appraised value of the

property (See 24 CFR 203.18(a)(3)).

Exception to the above limitation is allowed only where

the unit is covered by a HUD accepted insured ten-year

protection plan (List is in Appendix 10) described in

chapter 6 and the manufactured home has never been

occupied.  The application for insured financing must be

accompanied by the name of the Plan and a Builder's

Certification (See paragraph 3-3.) that the plans and

specifications for the property comply with all

standards and requirements specified herein.  Comply

with HUD Handbook 4930.3 for a "proposed construction"

property.

The Conditional Commitment/DE Statement of Appraised

Value, Form HUD 92800.5B, will be issued with a

condition requiring evidence to be submitted that

the property is covered by an acceptable Plan at the

time of the request for insurance endorsement.  In

this instance, the property could qualify for high

loan-to-value financing.  Borrower must be notified

by lender that Section 518(a) financial assistance

is not allowed.

3-5. PERMISSION TO START CONSTRUCTION (EARLY START) <TOP>.  This

procedure is appropriate to assist builders with starting

construction prior to the completion of the appraisal.  Use

of the "Early Start" will allow relief when backlogs occur

during high volume situations.  See Appendix 6 for a

suggested format.

After issuance of the building permit by the local authority

the mortgagee may request permission for the builder to

start construction before the issuance of a Conditional

2/92                         3-18

_____________________________________________________________________

4145.1 REV-2

CHANGE 1

(3-5.)

Commitment/DE Statement of Appraised Value, Form HUD

92800.5B.  An "Early Start Letter" issued by HUD or a Direct

Endorsement lender is deemed as an approval for mortgage

insurance before the beginning of construction (placement of

permanent construction on the site).

The builder must be on the approved certification list (see

paragraph 3-3.K.3)).  If project is borderline in character,

requiring complete processing to determine acceptability and

feasibility, the "early start" procedure cannot be used.

A.  Term of the Commitment - Normal term for proposed

construction cases, beginning with the date of the

"Early Start Letter".

B.  Mortgagee responsibilities.

1)  Comply with requirements listed in paragraph 3-3.D;

2)  Submit request to HUD field office;

3)  Submit a Builders Certification, Form HUD 92541, and

the Agreement to Execute a Builder's Warranty of

Completion of Construction, Form HUD 92541-A.  See

paragraph 3-3.E. for instructions); and

4)  Submit two copies of the plot plan.

C.  Architectural Branch Responsibilities.

Review the builders certification to determine

compliance with paragraph 3-3.A. through C.  Consult

with Valuation staff for any circumstances that would

prevent issuance of commitments.

1)  If acceptable, prepare a letter to the mortgagee

and/or builder similar to suggested format in

Appendix 6.  Place copy of letter in case binder.

2)  If unacceptable, notify the mortgagee.  Use Form HUD

92026, Report on Application.

3-19                 2/92

4145.1 REV-2

CHANGE 1

(3-5.)

D.  Builder Responsibilities.

1)  Construction exhibits must be available at site,

identical to those provided to the mortgagee;

2)  Call fee inspector to request initial inspection

(see paragraph 4-3); and

3)  Construct dwelling according to certified exhibits.

3-6. CONDITIONS OF COMMITMENT<TOP>.  The Conditional Commitment/DE

Statement of Appraised Value (Form HUD 92800.5B) contains

General Commitment Conditions and provisions for Specific

and Special Commitment Conditions.

A.  General Commitment Conditions apply to all cases for

compliance with HUD requirements and do not require

additional attention.

B.  Specific Commitment Conditions must be indicated by

checking the boxes applicable to the specific case.

C.  Special Conditions are extensions of the Specific

Conditions.  Provide to Direct Endorsement lenders for

their use.

The Architectural Special Condition Sheet (AC Sheet)

is attached to Conditional Commitment/DE Statement of

Appraised Value, Form HUD 92800-5B (see paragraph 3-7).

1). Developed by the Chief Architect, listing typical

special conditions for proposed construction cases.

List only those items that require compliance with

24 CFR 200.926d (See Appendix 8).

Provide a space for the FHA case number, the name of

the builder and the property address.  Allow enough

space for use when adding requirements that are not

preprinted on the sheet.  Review annually and revise

as necessary.

Number in sequence (i.e., AC-1, AC-2, etc.).  Do not

repeat general or specific commitment conditions

printed on the Conditional Commitment.

3-20

2/92

4145.1 REV-2

CHANGE 1

(3-6.C.)

2)  Disposition of AC Sheet.  Complete in triplicate.

Valuation Branch or DE Underwriter must include as a

Special Commitment Condition.

a.  Send the original to the mortgagee attached to

the Conditional Commitment/DE Statement of

Appraised Value, Form HUD 92800.5B;

b.  Retain one copy for the office case binder,

attach to Form HUD 92800.5B; and

c.  Place one copy in the "Inspection Copy" of the

construction exhibits.

3-7. FEE ALLOCATIONS FOR ARCHITECTURAL REVIEWS <TOP> are authorized in

field offices with severe staffing shortages in the

Architectural area.  Fee personnel must be knowledgeable in

the HUD Minimum Property Standard (24 CFR 200.926d) and the

exhibit requirements in chapter 2 of this handbook.

Fee processors must be in the locality of the field office

in order to ensure the timeliness of the review.  The cost

will not exceed fifteen dollars ($15.00) per case (Use Form

HUD 3650).  Variance from this fee must be obtained from

Headquarters, with adequate justification.

The review must only encompass the HUD requirements of

chapter 2 and 24 CFR 200.926d (See Appendix 8).  If a case

is rejected, the fee processor must fill out Form HUD 92026,

properly noting all non-compliances.  The field office is

responsible to spot check fee processors, work and review

all reconsiderations.  Fee processors, must be encouraged to

avoid rejecting a case when possible.

If the review is at the Post-Commitment/Post Endorsement

stage, complete Form HUD 92542 according to instructions in

paragraph 3-3.K.5).

Fees for Field Reviews are established by the field office;

however, the fee should be less than that of a final

inspection, because there are no photographs involved.  Fee

should also be based on interior and/or exterior inspection.

3-21                        2/92

4145.1 REV-2

CHANGE 1

3-8.  DESK REVIEW OF STAFF AND FEE PERSONNEL<TOP>.

The Chief Architect, Supervisor (or designee) must verify

the conclusions reported by fee or staff personnel.

A.  Items to be Reviewed:

1)  Basic cases in MCC (group) submissions (also review

yearly to check for compliance with changed

requirements);

2)  Cases in new areas that may establish precedents;

3)  Cases involving complex or unusual design features;

4)  Cases involving special programs in which the field

office has limited experience;

5)  Cases reviewed by staff personnel who are relatively

inexperienced in applying HUD procedures and

techniques;

6)  Cross section of cases for selective or spot check

review; and

7)  Inspection reports, including site grading and

drainage.

B.  Performance Review.  The work of each staff and fee

reviewer (10% minimum) and all inspection reports of fee

personnel will be office reviewed to assure accuracy,

consistency, good judgment and integrity.

Refer to Handbook 4020.1 for fee personnel requirements

for maintaining a good standing on assignments.

C.  Amendment Procedure.  The reviewer can amend or correct

any report prepared by staff or fee personnel.  All

modifications are noted, dated and initialed in red.

1)  Upon completion, the reviewer signs the case or

report to indicate any action and certifies that the

conclusions (as submitted or modified) are

justified.

2)  If the findings on the report are unacceptable, the

reviewer can prepare a new report.

2/92                         3-22

_____________________________________________________________________

4145.1 REV-2

CHANGE 1

3-9.  RECONSIDERATION PROCEDURE<TOP>.  Refer to:

A.  Handbook 4160.1, Reconsideration Before Endorsement.

B.  Handbook 4170.1, Reconsideration After Endorsement.

3-10. COMPLAINT PROCESSING PROCEDURES<TOP> (Refer to Handbook 4070.1

and 4000.4).

If the homeowner has a HUD accepted insured ten-year

protection plan, the HUD Field Office cannot get involved in

the complaint until all remedies are sought through the

warranty plan, including arbitration.  However, if a major

structural defect is involved, the Field Office can make a

site inspection and provide an opinion letter as to whether

a structural defect is present (or imminent).  This

documentation will be given to the homeowner to assist the

arbitrator in making a decision based on the merits of the

claim in lieu of obtaining information only from the Plan.

3-11. PROPERTIES TO BE REHABILITATED (203k program) <TOP>.  Follow

instructions in Handbook 4240.4 REV-1, dated September 1989.

3-12. SPECIAL MATERIALS, PRODUCTS AND METHODS OF CONSTRUCTION <TOP>.

When a proposed construction case involves a non-standard

material or product, refer to the applicable release or

bulletin (e.g., Materials Release, Use of Materials

Bulletin, Engineering Bulletin, Truss Connector Bulletin, or

a Mechanical Engineering Bulletin).

When a Structural Engineering Bulletin (SEB), Regional

Letter of Acceptance (RLA), or State Agency Regulation is

required, note as a Special Condition on Form HUD 92800-5B.

The field office will give the participant Handbook 4950.1,

Technical Suitability of Products Program, that describes

the review procedure when no Materials Release or Bulletin

has been issued.

3-13. PARTIALLY COMPLETED AND EXISTING CONSTRUCTION <TOP>.

The Valuation Branch is responsible for reviews and analyzes

existing properties (see Handbook 4905.1).  When requested,

Architectural staff will report on findings of any field or

office reviews concerning the acceptability under life and

safety requirements of HUD standards and applicable codes.

3-23                 2/92

4145.1 REV-2

CHANGE 1

3-14. CONDOMINIUM PROCESSING <TOP>.  See Handbooks 4150.1 and 4265.1.

3-15. REVIEW OF HUD ACQUIRED PROPERTIES <TOP> is the responsibility of

the Property Disposition Branch.  When requested, the

Architectural staff will work with the Valuation staff to

determine the best way to restore the properties to a good

condition and will estimate the cost of repairs.

When extensive rehabilitation is proposed, it may be

necessary for the Architectural Branch to prepare drawings

and specifications to permit competitive bidding on the

work.  Compliance inspections would also be provided.

3-16. PROCESSING WATER AND SEWERAGE SYSTEMS <TOP>.  This is done by

the Architectural Branch.  Advice should be requested from

HUD Environmental Engineer, Local Health Authorities, and

when necessary, HUD Central Office.  The following handbooks

establish criteria for the proposed systems:

A.  Central Water and Sewerage Systems, Handbook 4075.12.

B.  Minimum Design Standards for Community Water Supply

Systems, Handbook 4940.2.

C.  Minimum Design Standards for Community Sewerage Systems,

Handbook 4940.3.

3-17. RECORDS DISPOSITION <TOP>.  Comply with instructions in Handbook

2225.6, Appendix 20.  Construction exhibits must be

available for copying by any purchaser, homeowner or

warrantor as directed in Section 801 (Builders Warranty)

of the National Housing Act.

3-18. BUILDER'S WARRANTY <TOP>.  When the builder completes the Builder

Certification, Form HUD 92541, the builder must also sign

Form HUD 92541-A, Agreement to Execute a Builder's Warranty

of Completion of Construction (See Appendix 5).

On all properties that receive high loan-to-value financing,

the builder must complete (prior to closing the loan), Form

HUD 92544, Warranty of Completion of Construction and Form

HUD 92544-A, Addendum to Warranty of Completion of

2/92                          3-24

4145.1 REV-2

CHANGE 1

(3-18)

Construction (See Appendix 7).  These forms ensure that the

builder provides a one-year warranty on the property.

The Department provides a four-year structural defect

warranty on all "proposed construction" properties.  HUD

Handbook 4070.1, Chapter 2, explains the warranty; it was

established under Section 518(a) of the National Housing

Act.  If the builder is determined to be at fault for the

structural defect, the Department will allow the builder to

make repairs.  If the builder refuses to make repairs,

administrative sanctions will be imposed prior to the

Department providing financial assistance to the homeowner.

3-19. AFFIRMATIVE FAIR HOUSING MARKETING PLAN <TOP>.  See HUD Handbook

4000.2 and Handbook 8025.1.

3-25                 2/92

Architectural Processing and Inspections for Home Mortgage Insurance

Directive Number: 4145.1

4145.1 REV-2

# CHAPTER 4.  COMPLIANCE INSPECTIONS

4-1.   GENERAL <TOP>.  Compliance inspections are to ensure that the

mortgage risk in construction stays within acceptable

limits.  Refer to the Architectural Requirements Grid in

Appendix 11.  Inspections in no way relieve the builder of

his/her contractual obligations to the home buyer.

Commitments for insurance require the completion of

construction according to accepted drawings, any amendments,

description of materials and the conditions of the

commitment.  Construction must be in a manner equal to or

exceeding applicable HUD requirements.

HUD neither attempts to control the building operation nor

forces its requirements upon the builder.  However, HUD may

refuse to insure the mortgage if the provisions of the

commitment and all other pertinent requirements are not

fulfilled.

4-2.   INSPECTION RESPONSIBILITIES <TOP>.

A.  Architectural Staff:                                 See Para.

1)  Review performance of fee inspectors               4-18

on proposed construction cases                     4-24

2)  Field review five percent of all                   4-24

inspections performed by fee inspectors

3)  Manufactured housing inspections                   4-9

4)  Rehabilitation inspections                         4-11

5)  Truss fabrication inspections                      4-12

6)  Complaint inspections                              4-14

7)  Subdivision inspections                            4-15

8)  Casualty Damage inspections                        4-16

9)  Review mortgagee certifications                    4-25

___________________________________________________________________________

4-1                        3/90

___________________________________________________________________________

4145.1 REV-2

___________________________________________________________________________

(4-2.A.)

10) Review inspections made by                      4-26

the local authority

11) Train field office staff and                    4-27

fee personnel

12) Maintain adequate fee inspector panel           3-8.B

13) Provide architectural support to

other Branches in the Field Office

B.   Fee Inspector ensures that proposed construction is in

compliance with the local, State or CABO code and HUD

requirements in 24 CFR 200.926d (See Appendix 8), where

applicable:

For field review purposes, fee or staff inspectors

must submit to HUD Architectural Branch a signed,

legible copy of all compliance inspection reports

(new construction only).  Include photographs

discussed below.

1)  Substantial rehabilitation inspections

(See Handbook 4240.4)

2)  Proposed construction inspections (including

properties with a HUD accepted insured ten-year

protection plan)

a.  At final inspection (when all on-site

improvements have been acceptably completed),

fee inspectors (or fee appraisers, where

property is complete and has a ten-year

protection plan) are required to take a color

photograph of each diagonally opposite front and

rear corner of house to record adequate grading

and drainage of site.

Photographs should be taken from street at

the property corner (front shot) and from a

rear property corner (rear shot) diagonally

opposite the front property corner.

This also applies to final inspections

performed on all homes built under an

3/90                          4-2

4145.1 REV-2

(4-2.B.2)a.)

HUD accepted insured ten-year protection

plan.  See Appendix 8 for suggested format

for submitting photographs.

b.   Final inspection report must provide a specific

statement on acceptance of grading and

drainage.

Inadequate grading and drainage is the

biggest complaint of homebuyers and should

be carefully reviewed.  Also discuss

thoroughly in training sessions.

If reviewer notes questionable items in

photographs, amend compliance inspection

report accordingly and/or contact the

inspector.  A field review may be

necessary.

c.   Photographs must be permanently placed in case

binder.  Field Office must ensure that

photographs are included with the endorsed case

file sent to Headquarters.

d.   Where the first and second compliance inspection

are waived by the HUD Field Office (because the

local authority or a HUD accepted insured

10-year protection plan has been accepted to do

the inspections), the inspector must make the

following statement on Form HUD 92051:

"The initial and intermediate inspections

were waived because of __________________

(acceptance of inspections by the local

authority or the 10-year warranty by (name

of warranty company)).  The dwelling appears

to be completed in conformance with the

submitted construction exhibits."

3)  Repair inspections on existing properties are

completed according to instructions in paragraph

4-13.

Fee Appraisers are not to be assigned

inspections on proposed construction cases

unless they have been certified by the

4-3                         3/90

4145.1 REV-2

(4-2.B.2)d.3))

Architectural Branch to do fee inspections.

4)  Final inspections on properties "under construction"

or "existing less than one year old."  See paragraph

6-3.A.3).

4-3.  NUMBER OF INSPECTIONS <TOP>.  Not less than three inspections are

required on properties involving proposed construction cases

(See paragraph 4-4 for types), with the following

exceptions:

A.   Manufactured Housing.  Require an initial and final

inspection on:

1)  "Mobile" units conforming to Federal Manufactured

Home Construction and Safety Standard.  See

paragraph 3-4.

2)  "Modular" units conforming to a Structural

Engineering Bulletin, Regional Letter of Acceptance

or Category III State approval.  Refer to Handbook

4950.1 for criteria on plant inspection procedures.

B.   Properties Covered by Insured Ten-Year Protection Plans

may require only the final inspection (Refer to

paragraph 6-2 for acceptance criteria).  In areas where

specific problems are known to recur, or when there is a

high incidence of complaints against the builder,

the initial and intermediate inspections should be

required.  (Thoroughly document the Builder's file).

The builder must advise HUD of coverage prior to the

issuance of the conditional commitment for this

exception to apply or three inspections will be

required.

C.   Local Authorities that are approved by the HUD Field

Office can make the initial and framing inspections (See

paragraph 4-26).

Fee inspector must make the final inspection.

3/90                          4-4

4145.1 REV-2

(4-3)

D.  Additional inspections are warranted if:

1)   Builder is known for substandard work.  Continue to

inspect until quality of work and supervision is

acceptable.

2)   Builder is unfamiliar with HUD requirements.

3)   Proposal involves unusual construction methods or

site features.

4-4.   CONSTRUCTION STATUS NOTIFICATION TO THE INSPECTOR <TOP>.  Required

of the builder or mortgagee as indicated on Form HUD 92800-5B,

Conditional Commitment / Statement of Appraised Value.

After the HUD assignment of the fee inspector, the mortgagee

(or builder, if allowed by the lender) can directly contact

the fee inspector to schedule inspections.  If fee inspector

cannot he reached, call HUD Field Office for assignment of

another fee inspector (Construction exhibits must be

available at site).  Unless instructed otherwise, builder

can proceed with construction after giving proper notice.

If work is not complete and ready for inspection, fee

inspector may still charge a fee and require another

inspection when work is ready.

A.  Initial Inspection.  Required before the "beginning of

construction" (defined as placement of permanent

construction) with foundation forms in place.  Give two

workdays' notice.

An open excavation, without a minimum of foundation

forms in place, must be reinspected by the fee

inspector.

B.  Framing Inspection.  Required when the building is

enclosed and the framing, plumbing, heating, electrical,

and insulation is complete and visible.  Give one

workday notice.

4-5                         3/90

4145.1 REV-2

(4-4)

C.  Required Optional Inspections.  Required by a specific

condition of the commitment.  Give one workday notice.

Additional inspections may be required on

non-typical construction techniques, or when the

builders quality of workmanship is in question.

D.  Final Inspection.  Required when construction is

completed and property is ready for occupancy.  Give one

workday notice.

4-5.   PREMATURE CONSTRUCTION <TOP> is the beginning of construction

before HUD or the DE lender has issued a Conditional

Commitment/ DE Statement of Appraised Value, Form HUD

92800.5B, or an Early Start Letter.  Outstanding commitment

must be amended to reflect a reduced loan amount and term of

commitment ("existing" in lieu of "proposed" construction).

When the Compliance Inspection Report (Form HUD 92051) shows

premature construction, the reviewer judges the

reasonableness of the fee inspector's opinion from the date

of the report, the date of the commitment and the reported

progress of the work.  If the fee inspector's opinion

is not supported by factual evidence, reviewer modifies

report to show an acceptable start of construction.

A.  Property Eligibility.  Dwelling is no longer acceptable

for the maximum benefits provided by the National

Housing Act including the four-year structural defect

warranty under Section 518(a).

B.  The Mortgagee is Notified by letter (from Director of

Housing), with a copy of Compliance Inspection Report.

The remaining compliance inspections may be made.

4-6.   BUILDER REFUSAL TO UNCOVER CONCEALED CONSTRUCTION <TOP> goes on

the compliance inspection report in Part IV-A(b).  The Field

Office Manager must notify the Mortgagee by letter,

accompanied by the compliance inspection report.  This could

be a basis for refusal to endorse the loan and for a Limited

Denial of Participation (LDP) action against the builder.

3/90                            4-6

4145.1 REV-2

4-7.   POSTING HUD CASE NUMBER <TOP>.  Provided by the builder on each

property.  The number should be at least three inches high,

fade-resistant and easily read from the street.  If case

number has not been issued, use lot and block designation.

4-8.   POSTING EQUAL EMPLOYMENT OPPORTUNITY PLACARD (HUD 928.1) <TOP>.

Required by the Department of Labor, Office of Equal

Employment Opportunity.  HUD will furnish this to the

builder if they are available.

Display the placard in a conspicuous place at the job

site.  One placard may cover a group of contiguous

properties being constructed simultaneously by a

builder.

4-9.   MANUFACTURED HOUSING INSPECTIONS <TOP>.

A.   Mobile.  See instructions in paragraph 3-4.

B.   Modular.  Factory built units must comply with the

applicable code and with 24 CFR 200.926d.  See Handbook

4950.1 and Appendix 8.

4-10.  DIRECT ENDORSEMENT INSPECTIONS <TOP>.  See Handbook 4000.4.

4-11.  REHABILITATION INSPECTIONS <TOP>.  See Handbook 4240.4.

4-12.  TRUSS FABRICATION INSPECTIONS <TOP>.  The manufacturer is subject

to periodic inspection by HUD Field Office.  Inspections

apply to the manufacture of finished products and individual

components.  Truss connectors, fabrication and materials

must be carefully checked.

If  structural deficiencies exist, report non-compliance

to the Field Office Manager, who forwards the findings

to the Deputy Assistant Secretary for Single Family

Housing.

4-7                          3/90

4145.1 REV-2

4-13.  REPAIR INSPECTIONS <TOP>may be assigned to a fee inspector by the

Valuation Branch; however, it is the responsibility of the

Valuation Branch to review the reports and do the field

reviews.  Only a completion inspection is required.  The

inspection is paid by the mortgagee.

Major deficiencies that are not commitment conditions

are described by memorandum to the Valuation Branch with

recommendations for correcting the problem.

Non-commitment items are not to be included on the

compliance inspection report.

4-14.  COMPLAINT AND STRUCTURAL DEFECT INSPECTIONS <TOP>.  See Handbook

4070.1, Handbook 4000.4 and paragraph 3-10 of this

handbook.  Also see paragraph 6-2.F.4).

4-15.  SUBDIVISION INSPECTIONS <TOP>.  See Handbook 4135.1.

4-16.  CASUALTY DAMAGE INSPECTIONS<TOP>.  See Handbook 4330.1.

Mortgagees are to contact and pay fee inspectors directly.

Fee inspectors report findings on Form HUD 92051 and return

it to the mortgagee.

4-17.  ROTATION OF INSPECTORS <TOP>is recommended within subdivisions or

communities with a large volume of construction.  In

subdivisions using Master Conditional Commitments or Master

Appraisal Reports, inspectors should be rotated every six

months.

The same fee inspector must not make more than four

inspections on a single property.  However, if four or

more inspections are required on Form HUD 92800.5B, then

one additional inspection will be allowed for the same

inspector.  If only the final inspection is required on

a property (because the builder is providing a HUD

accepted 10-year protection plan), then one additional

inspection will be allowed for the same inspector.

4-8                          3/90

4145.1 REV-2

4-18.  OFFICE REVIEW OF INSPECTION REPORTS (Form HUD 92051) <TOP>.

Follow the instructions in Handbook 4190.1. Also see

paragraph 4-2.B.1).

Reports may reveal need for field review, special

handling of a case or the need for added special

construction requirements to satisfy applicable code(s),

or HUD's Minimum Property Standards.

4-19.  ISSUANCE OF INSPECTION REPORTS <TOP>.  The initial and

intermediate (framing) inspection can be considered official

when signed by fee inspector.  Chief Architect must

determine the extent of this policy.  Continued

issuance of reports without office review must depend on

reasonable assurance of quality inspections.

Exception:  All repair and all final inspection reports

must be reviewed before becoming official, signed by the

Chief Architect or designee.

4-20.  UNREPORTED NON-COMPLIANCE <TOP>.  If the noted deficiency was

probably visible at a previous inspection it should be

reviewed to determine whether corrections are essential.

An inspection report made at any stage of construction

(except final inspection) should be construed as final

acceptance of work previously performed.

4-21.  EXTENSIVE NON-COMPLIANCES <TOP> are (1) unacceptable variations

to the HUD-accepted exhibits or; (2) unacceptable

construction.

Before checking box 11, Form HUD 92051 (& appropriate

subboxes), the fee inspector should conclude that

correcting the non-compliance would be impracticable and

the property would be unacceptable under the outstanding

conditional commitment, or builder refusal to correct

the problem is evident.

4-9                         3/90

4145.1 REV-2

4-22.  CONSTRUCTION CHANGES <TOP> are revisions to the accepted set of

exhibits.  Provide to HUD in one of three ways:

A.   Mortgagee Submits Form HUD 92577, Request for Acceptance

of Changes in Approved Drawings and Specifications

(Instructions are in Handbook 4190.1).  If forms are not

available, the mortgagee may submit a letter.  Ensure

that the essential information contained on the form is

present.

Fee inspector may review Form HUD 92577 and make

recommendations and comments at the time of inspection.

B.   List Changes on Compliance Inspection Report.  Fee

inspector may encounter changes during inspection and

should report changes on Form HUD 92051.  Field Office

may require the mortgagee to submit Form HUD 92577.

C.   Field Acceptance of Changes.  Fee inspector can accept

only minor "no cost/non-structural" variations.  Do not

report them on Form HUD 92051.

4-23.  ACCEPTANCE OF CONSTRUCTION CHANGE(S) <TOP> is official when the

mortgagee has been properly notified in writing by the HUD

Field Office Manager or Supervisor by issuance of a

countersigned Compliance Inspection Report or Request for

Acceptance of Changes.  The builder assumes the risk for

non-approved changes.

Any change from the approved construction exhibits may

be the subject of a homeowner complaint and should be

viewed in this light by the fee inspector.

When proposed changes have been accepted by HUD or the

DE lender, the exhibits are considered as having been

amended.  Send copy to fee inspector.

If an accepted change is not made, the fee inspector

must report the variation from the amended exhibits.

A.   Minor variations are departures from the approved

construction exhibits whose identification on the

3/90                            4-10

4145.1 REV-2

(4-23.A.)

Compliance Inspection Report would serve no useful

purpose.

1)  Cost limits should not be established and used as a

criterion (e.g., elimination of a five-dollar light

fixture or twenty (20) dollar exhaust fan might be

unacceptable).

2)  If there is any doubt about acceptability, the

inspector must report the change (e.g., Relocation

of a window may be considered a minor variation;

however, if it adversely affects the dwelling, the

change should be reported).

B.  Fee Inspector Responsibilities.  Review the change at

the time of inspection to ensure that the changes are

appropriate, technically sound, adequately described on

Form HUD 92577 and that the form has been properly

completed.

1)  Report all variations that have not been processed

under paragraph 4-22.C. or previously accepted by

HUD.

2)  Describe change(s) on the Compliance Inspection

Report, but do not show acceptance of any variation

even though it appears to be technically acceptable.

Replacement cost and mortgage terms could be

affected.

3)  Differentiate between acceptable minor variations

and those changes that are not acceptable for field

approval.  Plan reversals must be reported.

4)  Review the cost estimate and ensure that it is

reasonable.  Modify if necessary.

4-11                      3/90

4145.1 REV-2

4-24.  FIELD REVIEWS <TOP> are required on at least five (5) percent of

all inspections and will be reviewed by the Chief Architect

(Supervisor or designee).  Use Form FHA HUD 1038-A,

Architectural Field Review Report.  Make specific comments

on grading and drainage (See Appendix 8 and paragraph

4-2.B.).  Report monthly to the records clerk.

Chief Architects must keep informed of the general quality

of inspections and fee inspector performance.  In areas

where local authorities do the initial and intermediate

inspections and in areas where these inspections have been

waived because the builder has a HUD accepted insured

ten-year protection plan, all phases of construction should

be field-reviewed semi-annually to determine continued

acceptability of local authority inspections.

In areas where a lender has been approved to submit

applications to a nearer office (Lender's Option), it is the

responsibility of the office that processed the loan to make

the field reviews on the property.

4-25.  REVIEW OF MORTGAGEE CERTIFICATIONS <TOP>.  Architectural Branch is

responsible for reviewing at least ten (10) percent of all

certifications on "proposed" and "existing" properties.

A.  Field Review at least five (5) percent for reliability.

1)  Use Compliance Inspection Report, Form HUD 92051.

2)  Mortgagees must be notified if corrections are

necessary for a clear certification.  If improper

certification is apparent, request lender to make

repair out of their own funds.

B.  Maintain a Record of Certifiers.  If mortgagees provide

unacceptable certifications, referral to Mortgagee

Approval Board may be necessary.

4-26.  INSPECTIONS BY LOCAL AUTHORITIES <TOP> can be approved by the HUD

Field Office for the initial and intermediate stages of

construction.  The builder should give a copy of the

inspection record (or certificate of occupancy) to the fee

inspector at final inspection, clearly identifying the

3/90                        4-12

4145.1 REV-2

(4-26.)

property (Also see paragraph 4-2.B.2)d.).  Maintain a record

of this approval; include copies of field reviews to verify

continuing acceptance.

Just because a local authority is acceptable to do

subdivision processing under the Local Area

Certification process is no reason to automatically

approve them to do the initial and intermediate

inspections.

A.  Approval criteria.

1)  Local authority determines that each dwelling has a

continuing supply of safe and potable water,

sanitary facilities, sewage disposal and surface

water disposal to ensure healthful conditions;

2)  Local inspections determine that footings or

ground-supported slabs will rest on soil that is

stable and has an acceptable bearing capacity;

3)  Local inspections determine that footings are

suitable to support the anticipated loads and are

below the locally prevailing frost line;

4)  Local inspections assure structural safety and

safety from fire hazards;

5)  Local inspections are made on electrical and

plumbing work to ensure safety, durability and

satisfactory performance; and

6)  Local authority provides the builder with tags,

slips or other means of recording required

inspections.

B.  Substandard Construction.  The HUD Manager must notify

the local authority an builders when HUD compliance

inspections may resume.

4-13                         3/90

4145.1 REV-2

4-27.  TRAINING of HUD Field Office <TOP> staffs and fee personnel is the

responsibility of the Chief Architect (Supervisor) to

improve the quality and efficiency in accepting, for

processing and inspection purposes, the architectural

exhibits and builders' certifications.

A.  Training Sessions should be scheduled semi-annually;

however, one a year is mandatory.  When distance from

the HUD Field Office is a problem, fee personnel should

be kept advised of developments and attend when

practicable.

1)   Discuss grading and drainage problems.  Use field

review notes and examples of photographs (See

Appendix 8 and paragraph 4-2.B);

2)   Provide information on new materials and unusual

details of construction;

3)   Train personnel to inspect work of various trades;

and

4)   Personnel should be encouraged to offer suggestions

and help solve problems.

B.  Inspection Guide must be prepared by each HUD Field

Office for issuance to inspectors and staff personnel.

See paragraph 7-3.

4-28.  HUD RECOMMENDATIONS FOR MODEL CODE CHANGE(S) <TOP>.

Suggested by staff personnel to correct errors, omissions,

or to make improvements to the content of a model code.

Interpretations of model codes will be by their publishers.

A.  Submit Proposals to Headquarters, Office of Manufactured

Housing and Regulatory Functions, Construction Standards

Branch.

B.  Justification for Change.  Each proposal must include

supporting statements for the change and specifically

3/90                          4-14

4145.1 REV-2

(4-28.B.)

identify:

1)  Clearly state the purpose of the proposed code

change to:

a.  Clarify the code, or revise outdated material;

b.  Substitute new or revised material for current

provisions of the code;

c.  Add new requirements, etc.

2)  Provide reasons for changing the current provisions

of the code.  Changes that add or delete requirements

must be supported by logical arguments that clearly

show why the present code provisions are inadequate

or overly restrictive, and how the change will

improve the, code.

3)  Include all available substantiating material, i.e.

(but not limited to) research reports, statistical

analysis, field-related experiences and ease or

difficulty in enforcing or interpreting the code.

When possible, technical data should be provided to

substantiate the need for a code change.  Submit

copy of such published data or indicate where to

obtain.

4-15                         3/90

Architectural Processing and Inspections for Home Mortgage Insurance

Directive Number: 4145.1

4145.1 REV-2

# CHAPTER 5 - ESCROW PROCEDURES

5-1.   GENERAL <TOP>.  Escrow requests on proposed and existing cases

are reviewed by the Architectural Branch when

extenuating circumstances (beyond contractor's control)

warrant acceptance of delayed completion items.  Direct

Endorsement (DE) Lenders are responsible for approval of

escrows; however, after closing documents are submitted,

HUD Field Office will monitor completion.  The following

methods are provided:

A.   Mortgagee's Assurance of Completion (Use Form HUD

92300, for deferred on-site and limited off-site

work).

B.   Escrow Agreement for Postponed Off-Site Improvements

(For deferred off-site work only, use Form HUD

92606).

C.   Assurance of Completion by a Public Authority (For

deferred off-site work only).

An assurance of completion using either Forms HUD 92300

or 92606 will be supported by an escrow of cash or an

irrevocable commercial letter of credit (Bonds are

unacceptable).

5-2.   MORTGAGEE'S ASSURANCE OF COMPLETION (Form HUD 92300) <TOP>.

Submit in HUD case binder according to instructions on

form and in Handbook 4190.1 and 4000.4.  Prepare escrow

only after issuance of clear final inspection report

(Form HUD 92051) by the fee inspector.

A.   Conditions of Use.  The following must prevail for a

clear final inspection:

1)  The record of the mortgagee for fulfilling

previous assurances of completion must warrant

approval of submitted form.

2)  The dwelling on the mortgaged premises is

habitable, safe, and essentially complete;

Items  essential for customary occupant use

and enjoyment, or for property safety or

durability, may not be escrowed.

5-1                         3/90

4145.1 REV-2

(5-2.A.)

3)  The deferred work cannot be acceptably completed

in a reasonable period before closing, due to

weather or other extenuating circumstances; but

there is assurance it can be completed within no

more than eight months.

Completion cannot be later than the date

established on Compliance Inspection Report,

Form HUD 92051;

4)  The work involved in the escrow arrangement is

minor and uncomplicated; and

5)  All other conditions of the commitment,

including specific conditions not subject to

field inspection, have been fulfilled by

compliance or by an acceptable assurance of

completion (i.e., off-site improvements).

B.  Instructions for Completing Form HUD 92300 are on

the form and in the Single Family Underwriting

Reports and Forms Catalog (Handbook 4190.1).

C.  Minimum Amount of Cash/Irrevocable Letter of Credit

Requirement is $500 or 1-1/2 times the cost of the

deferred item, whichever is greater.

D.  Mortgagee's Responsibilities (HUD Processed):

1)  Provide written request for all escrows to HUD

Field Office.  DE lender must get written

request from builder; and

2)  Assure completion of the deferred work.  If the

escrow proves insufficient, the lender must pay

for the deficit; and

3)  Call fee inspector; obtain a Compliance

Inspection Report (Form HUD 92051).

E.  Extension of the Completion Date is allowed when

Office Manager (or Director of Housing) determines

that the deferred work could not have been

3/90                              5-2

4145.1 REV-2

(5-2.E.)

completed within the established time frame.  When

an extension is granted, the tickler file date is

amended.

F.  Receiving Clerk Responsibility is to forward all

escrow requests to the Architectural Branch for

disposition.

G.  Architectural Branch Responsibility is to review all

escrow requests for HUD processed cases.  Consult

with the Valuation Branch, if necessary, on existing

properties.  DE underwriter is responsible to review

all escrows on DE cases.

1)  If acceptable, prepare an office report on Form

HUD 92051, Compliance Inspection Report, marking

Item 13 - Part I.

a.  Provide a description of the acceptable

escrow item(s) in Part II of form; include

cost estimate of each item;

b.  Provide the name and phone number of a fee

inspector for the mortgagee to contact upon

completion of escrow item(s) or mark Item 12

- Part I, allowing the mortgagee to

certify;

c.  Provide the total amount of the escrow and

the date of completion in Item B - Part IV;

d.  Place a stamp "92300 Escrow Acceptable"

(3/8" high letters) on the front cover of

the case binder.

2)  If unacceptable, note such on mortgagee's

request; initial and date it; then return to the

mortgagee.

H.  Closing Clerk Responsibility:

1)  Review Form HUD 92300 for completeness.

5-3                        3/90

4145.1 REV-2

(5-2.H.)

2)  If completion date shown on the form has passed,

do not issue Mortgage Insurance Certificate

unless the Director of Housing (Supervisor, or

designee) grants an extension, in writing.

3)  Enter "92300 Escrow" and the escrow amount

directly below endorsement stamp on the note.

4)  Prepare a supplementary binder showing "FHA case

number", followed by "92300 Escrow".  Place

Parts 1 and 2 of Form HUD 92300 inside the

binder.

5) Deliver the binder to the Architectural branch.

I.   Monitoring Function.  Architectural Branch is

responsible on all proposed and existing property

escrows, including those accepted by DE Lenders.

1)  At least 5 percent of the mortgagee assurances

of completion must be field reviewed to

determine reliability.

2)  Maintain a list of acceptable certifiers

(mortgagees) as well as those certifiers making

false or otherwise unacceptable certifications.

When administrative sanctions are necessary,

follow HUD Handbook 4060.2.

3)  After receipt of the supplementary binder, set

up a tickler card file to trigger the mailing of

Part 2 (Form HUD 92300) 15 days before the

designated completion date.  When an extension

is granted, the tickler card is revised to

reflect the new date.

a. When Part 4 of Form HUD 92300, and Form HUD

92051 is received from the lender, then:

1.  Pull the supplementary binder.

3/90                              5-4

4145.1 REV-2

(5-2.I.3)a.2.)

2.  Remove the original Part 1 (Form HUD

92300), staple it to the certified

Part 4, with Form HUD 92051.

3.  Mail the forms to Central Files in

Headquarters.  Instructions are in

Handbook 4165.1.

4.  Destroy the supplementary binder.

b.   When tickler date is reached and Part 4 of

Form HUD 92300 and Form HUD 92051,

Compliance Inspection Report, have not

been received from the mortgagee, send Part

2 of the form to the mortgagee as a

reminder.

The tickler card is refiled with a 30 day

due date.

c.   If Part 4 is not returned with the required

certification before the second date has

passed, prepare a letter to the mortgagee

for the Director's signature stating that:

1.  The mortgagee may not use the HUD escrow

procedure in the future until the

subject case deficiency is resolved.

2.  If deferred work is not completed within

30 days, the case will he referred to

the Mortgagee Participation and

Compliance Division in Headquarters.

3.  If the property goes into foreclosure,

the claim will be reduced by the amount

of escrow established for any incomplete

work.

J.  Properties Acquired by HUD before completion of

escrow procedure will be handled according to repair

instructions in Handbook 4310.5.  The insurance

claim will be reduced accordingly.

5-5                        3/90

4145.1 REV-2

(5-2.)

K.    Revision of Deferred Completion Items.

1)  Mortgagee (including DE lender) submits request

for waiver or substitution of required work

items.  Mortgagor approval must be provided.

2)  Architectural Branch consults with Valuation to

consider acceptability of property changes and

effect, if any, of proposed changes on value.

3)  If approved, and value is adversely affected,

all or any excess of escrow funds must be

applied as a mandatory prepayment to mortgage

principal.

4)  Notify mortgagee of decision, including

prepayment requirements, with a copy to Central

Records in Headquarters.

5-3.    ESCROW AGREEMENT FOR POSTPONED OFF-SITE IMPROVEMENTS <TOP>.

Instructions are in Handbook 4115.1, Administrative

Instructions and Procedures, Chapter 3.  Use Form HUD

92606.

5-4.    ASSURANCE OF COMPLETION BY A PUBLIC AUTHORITY <TOP>.

Instructions are in Handbook 4135.1, Procedures for

Approval of Single Family Proposed Construction

Applications in New Subdivisions.

3/90                            5-6

Architectural Processing and Inspections for Home Mortgage Insurance

Directive Number: 4145.1

4145.1 REV-2

CHANGE 1

# CHAPTER 6 - INSURED TEN-YEAR PROTECTION PLAN

6-1. GENERAL <TOP>.  The requirements for an insured ten-year

protection plan (Plan) accepted by the Department are in

24 CFR 203.200 through 203.209.

For cases where the HUD Field Office determines the property

will be covered by a HUD accepted insured ten-year

protection plan, only the final inspection is required.

However, this does not preclude the HUD Field Office from

making additional inspections at any time.  It allows the

Field Office to reduce the number of inspections to one, a

"final inspection."  In this case a Certificate of Occupancy

or similar approval from the local jurisdiction is required.

Builders whose quality of construction has not been

demonstrated, or builders who have a record of construction

complaints will be required to obtain all inspections from

HUD, as outlined in paragraph 4-4.  When all inspections are

required, the HUD Field Office will notify the Plan in which

the builder participates.

"Insured ten-year protection plan (Plan)" means an

agreement between a homeowner and a Plan Issuer which,

among other things, contains warranties regarding the

construction and structural integrity of the

homeowner's one-to-four-family dwelling covered by an

FHA insured mortgage.  A Plan Issuer may be a State, an

insurance company, a warranty company, a Risk Retention

Group, a builder, or any other HUD-approved entity with

the required insurance backing.

While the vast majority of defects and claims can be readily

assessed to determine warranty coverage, unusual

circumstances sometimes arise that do not always lend

themselves to an obvious answer.  While an arbitration

process exists to normally resolve such disputes and the

Department could sustain a financial loss regarding the

outcome of such a dispute, the Office of the Deputy

Assistant Secretary for Single Family Housing expects HUD

Field Offices to provide Headquarters notification of such

disputes upon learning of such a problem.

6-1                2/92

4145.1 REV-2

CHANGE 1

(6-1.)

In unusual situations, Headquarters may decide to review the

circumstances surrounding the claim.  The Field Office may

be requested to provide information relating to the nature

and severity of the actual physical damage to a property,

the extent to which further engineering tests identified

additional unseen damage, the anticipated costs to restore

the structural member, etc.  Such information will be

reviewed and a position adopted as to whether a structural

defect exists under the criteria specified in HUD's

regulations.  Headquarters will advise the homeowner and the

warranty provider of its decision.

The term "proposed construction" is defined as "A property

approved for mortgage insurance or loan guaranty prior to

the beginning of construction and inspected during the

construction period by HUD-FHA or the Department of Veterans

Affairs (VA) (Refer to HUD Handbook 4070.1).

Refer to the Architectural Requirements Grid in Appendix 11.

A borrower may obtain a maximum loan-to-value ratio mortgage

(minimum downpayment) in cases involving proposed

construction.  In such cases the borrower may, if

circumstances warrant, file an application to HUD (within

four years from endorsement of the mortgage) for assistance

to correct structural defects under Section 518(a) of the

National Housing Act (See Handbook 4070.1).

A borrower will not be considered ineligible for Section

518(a) assistance merely because the borrower chooses to

increase the downpayment amount.  However, a borrower will

be ineligible for assistance if construction was started

prior to HUD's issuance of a Conditional Commitment (or

early start letter) or the issuance of a Statement of

Appraised Value from a DE lender.  Similarly, if VA

processing is used by the mortgagee, a borrower will be

deemed ineligible for 518(a) assistance if construction was

started prior to VA's issuance of a Certificate of

Reasonable Value (or early start letter).

A borrower may obtain a maximum loan-to-value ratio mortgage

if the builder began construction prior to the issuance of a

conditional commitment and a HUD accepted insured 10-year

protection plan is provided to the borrower.  However, since

construction was already started, item 16 of the specific

2/92                         6-2

4145.1 REV-2

CHANGE 1

(6-1.)

conditions on the back of Form HUD 92800.5B would be

applicable, stating:

"Borrower Notification:  At the time of the application

for Firm Commitment, the mortgagee must include a

certificate that the borrower was notified that the

property was not approved for mortgage insurance prior

to the start of construction and the Department does

not have authority to provide financial relief for any

future property repairs under Section 518(a) of the

National Housing Act."

6-2. ACCEPTANCE CRITERIA <TOP>.  Requests for initial HUD acceptance or

renewal of acceptance of a Plan should be made to the Deputy

Assistant Secretary for Single Family Housing, Department of

Housing and Urban Development, 451 Seventh Street, S.W.,

Washington, D.C. 20410.  Requests must be accompanied by

information and documentation evidencing Plan compliance

with the provisions in this chapter.  Applicants will be

notified of the Department's determination to accept or

reject their Plan.  If a Plan is rejected, the applicant

will be advised of the reason for rejection.  A list of the

acceptable plans is in Appendix 10.

The applicant may appeal the rejection to the Assistant

Secretary for Housing, at the above address, stating

specifically why the Plan should be approved.  The Assistant

Secretary (whose decision is final) will, within a

reasonable time, advise the applicant whether the rejection

will be upheld or reversed.

Plans may be issued (1) by a builder, warranty company,

insurance company, or a Risk Retention Group; or (2) by a

State that guarantees the builder's performance and the

State's continuing financial backing throughout the Plan's

coverage period.  The functions of a Plan issuer and an

insurance backer may be performed by a single corporate

entity.

All Plans must have insurance backing unless backed by the

full faith and credit of a State, in compliance with the

requirements in this chapter.  HUD will not accept Plans

backed by a State agency or a State insurance guaranty fund

unless HUD is assured that the full faith and credit of the

State is pledged to satisfy any and all obligations of the

State agency or guaranty fund that may arise in connection

with its financial backing of a Plan.  Where a State has a

6-3                 2/92

4145.1 REV-2

CHANGE 1

(6-2.)

home protection act or other statutes or regulations that

require its approval of Plans, a Plan Issuer must

demonstrate such approval to HUD as an additional

prerequisite to HUD acceptance.

"Insurance backing" means the direct insurance or

reinsurance of potential Plan obligations by one or

more insurance companies.

A.   Plan Acceptability.  HUD must be assured that:

1)   If a builder fails to correct structural defects

or construction deficiencies in a property covered

by a Plan during the term of any warranty offered

by the builder on the property, the Plan Issuer

will effect the corrections in accordance with the

terms of the Plan; and

2)   If a Plan Issuer fails to correct the deficiencies

or defects, or otherwise fails to honor the terms

of its coverage, its insurance backer or, if the

Plan Issuer is an insurance company, the insurance

company itself, will effect the corrections or

otherwise honor the Plans terms.

B.   Expiration of Plan Acceptance.  Unless renewed, Plan

acceptance by HUD expires automatically on the second

anniversary date of acceptance.  Plan acceptance by HUD

will be for a two-year period.  Insofar as practicable,

HUD will respond to a Plan Issuer's request for

acceptance of a change within 30 days of receipt.

1)   The Plan Issuer must apply for acceptance renewal

at least two months, but no more than three

months, in advance of expiration to avoid

automatic acceptance termination.

a.   Plan Issuer must submit a written

certification, no later than three weeks

before the anniversary date of the Plan's

acceptance by HUD, that the insurance company

backing its Plan is still an insurance

carrier approved by the State insurance

commission (or the equivalent entity) in each

2/92                         6-4

_____________________________________________________________________

4145.1 REV-2

CHANGE 1

(6-2.B.1)a.)

jurisdiction in which the Plan is offered.

Any company operating as a Risk Retention

Group will be regarded as having met

licensing, filing and approval requirements

of all states provided the Plan demonstrates

that it (1) meets licensing, filing and

approval requirements in its domicile state;

(2) meets each of the requirements of Section

(a)(4) of the Liability Risk Retention Act,

paragraphs (A) through (H).

b.   Plan Issurer must submit a written

certification, no later than three weeks

before the anniversary date of the Plan's

acceptance by HUD, which sets forth the

states in which the Plan operates.

c.   Plan Issuer must submit a listing of claims,

repairs and awards made on FHA insured

properties.  The first list is due no later

than September 1. 1992, and on an annual

basis thereafter.  Provide the following

information on an individual and accumulative

basis: (1) Plan Issuer's enrollment number;

(2) FHA case number; (3) Homeowner's name;

(4) Property address; (5) Effective date of

enrollment; (6) Date the claim was received;

(7) Date the claim was closed; (8) Number of

days to resolve claim; (9) Amount of award;

and (10) Description of claim.

Plan Issuers enrollment and claim forms must

include a space showing whether the property

is FHA insured (or other); another space is

required for the FHA case number, when

applicable.

d.   Plan Issuer must provide a Quality Control

Plan by April 15, 1992, that is acceptable to

the Secretary describing how the Plan: (1)

approves a builder; (2) assures the proper

inspection of a property; (3) field reviews

the quality of inspections; (4) reviews the

construction quality of the builder; (5)

terminates a relationship with a builder; (6)

processes homeowner complaints and

6-5                    2/92

4145.1 REV-2

CHANGE 1

(6-2.B.1)d.)        arbitration proceedings; and (7) notifies HUD

of any problems with the above.  In addition,

the Plan must provide a homeowner information

pamphlet describing in "plain English" the

(a) insurance provisions of their Plan; (2)

complaint handling procedures; and (3) the

procedures required for arbitration.

2)   Acceptance of a Plan will be continued beyond the

date of automatic termination only by a written

notification to the Plan Issuer by HUD and only if

the delay is caused by a lack of timely HUD

processing of a renewal application.

3)   HUD will not extend the expiration date of a prior

Plan acceptance if the Plan Issuer has negligently

provided incomplete information with its renewal

application.

4)   After a Plan has been accepted by HUD, there will

be no change in, or modification to, the Plan

coverage or the obligations of the homeowner under

the Plan or in its insurance backers or insurance

contract(s), without prior written HUD acceptance

of such change or modification, except changes

mandated by other applicable laws may not require

HUD's prior approval.  written notification to HUD

with adequate documentation will be necessary,

prior to the change taking affect.

A violation of this condition may be cause for

termination of a Plan's acceptance, and may be

grounds for initiation of sanctions against the

Plan Issuer in accordance with 24 CFR Part 24.

C.    Termination of Plan Acceptance.  The Department will

take into consideration such reliable evidence as is

made available to the Department of a Plan Issuer's

failure to fulfill its obligations.

Where HUD has credible evidence of a Plan Issuer's

failure to correct covered homeowner problems, or there

are justifiable homeowner complaints about untimely

problem resolution by a Plan Issuer, HUD will consider

this as cause for termination of a Plan's acceptance

and as grounds for initiation of sanctions against a

2/92                            6-6

4145.1 REV-2

CHANGE 1

(6-2.C.)

Plan Issuer or insurance backer in accordance with 24

CFR Part 24.  The Department will always look to the

Plan Issuer if their insurance backer does not properly

satisfy the intent of the complaint process.

Under no circumstances can a Plan Issuer require the

surrender of the insurance policy without the express

written consent by the Department except where the

homeowner has in the aggregate received full payment

for the face amount of coverage provided by the policy

or Certificate of Enrollment.  If a Plan accepts the

surrender of the policy without HUD's consent, the Plan

will still be liable for any damage that may occur to

the home, throughout the warranty period.

Field Offices must submit documentation to support any

recommendation for Plan termination to the Deputy

Assistant Secretary for Single Family Housing.  If the

Department proposes to terminate a Plan's acceptance,

Plan Issuer will be advised in writing of the

reason(s).  Procedures in 24 CFR Part 24 will apply.

D.   Insurance Coverage.  The protection plan must be backed

by an insurance company that is licensed and approved

to offer that coverage by the proper State Regulatory

Agency in each State in which the plan will operate.

Any company operating under the Product Liability Risk

Retention Act of 1981 (The Act), as amended, will be

regarded as having met licensing, filing, and approval

requirements of all States, provided the Plan

demonstrates that it (1) meets licensing, filing and

approval requirements in its domiciliary State; and (2)

meets each of the requirements of Section (a)(4) of the

Act, paragraphs (A) through (H).

1)   Warranty document.  A Plan Issuer must provide

homeowners an executed coverage contract clearly

describing the following:

a.   Identity of the property covered;

b.   The time at which coverage begins.  Plan

coverage must take effect at closing or

settlement following the initial sale of the

6-7                 2/92

4145.1 REV-2

CHANGE 1

(6-2.D.1)b.)        property to the homeowner.

Insurance does not begin when the builder

rents the property to a potential buyer; but

only after the closing of the loan occurs and

the first homebuyer takes possession.  The

home is considered complete at the time of

settlement of the loan to the homebuyer and

in no event later than date of the FHA

endorsement Of the mortgage;

c.   Maximum amount of Plan liability;

d.   Non-cancellable by the Plan Insurer or its

insurance backer;

e.   Transferability.  Coverage must automatically

transfer to subsequent owners without any

additional cost;

f.   The property coverage provided;

g.   Any exclusions from coverage;

h.   Performance standards for resolving homeowner

complaints and claims.

Standards must be fair, reasonable and

consistent with the intent of the Plan,

including the requirements in this chapter.

The standards must be acceptable to HUD.

i.   Dispute settlement procedures;

j.   Disclosure to homeowner on the ability to

appeal an arbitrators decision to HUD.

The disclosure must be of such "type size" to

be easily readable by the homeowner.

k.   Names, addresses and telephone numbers of the

Plan Issuer and its insurance backers; and

l.   When, to whom, under what conditions, and to

what address homeowners should submit any

construction deficiency complaint or

structural defect claim.

6-8                 2/92

4145.1 REV-2

CHANGE 1

(6-2.D.)

2)   Cost of Plan.  Must be prepaid by the builder, or

the Plan Issuer must provide irrevocable coverage

to the lender and purchaser at the time of

settlement.  The coverage documents may be

provided to the homeowner in specimen form at the

time of settlement with the actual documents to be

delivered thereafter in a four to six week

timeframe.  The cost of any optional coverage that

is in addition to what is required by HUD may be

paid by either the builder or the homebuyer.

3)   Payments under a Plan.  If a Plan Issuer or

insurance backer elects to compensate a homeowner

for damage to the homeowner's property that is

covered under a Plan in lieu of the Plan issuer's

making repairs, any such payment must be made

jointly to the mortgagee and the homeowner.

Mortgagees are responsible to ensure that the

property continues to be in a liveable condition

and the property does not become a foreclosure

risk to the Department as a result of known

defects.  Endorsement of the check by the

mortgagee will signify its acceptance of the

settlement offer by the Plan Issuer.  The

repair(s) should be completed; however, in cases

where it is determined that repair is not

warranted, the mortgagee must apply the

compensation in reduction of the outstanding

indebtedness of the mortgage.

Settlement offers must be in writing and the

homeowner given a minimum of 10 workdays to

respond.  Settlement offers over $5,000 must be

reviewed onsite by a HUD approved fee inspector

(inspection costs to be paid by Plan Issuer)

except: (1) where the settlement offer is made

pursuant to a binding bid by an independent third

party contractor, or (2) where payment is made to

the homeowner in settlement of legal action, or

(3) where the homeowner is represented by legal

counsel.

The homeowner must be able to award a contract to

the independent third party contractor that

submitted the binding bid to the Plan Issuer.

6-9                 2/92

4145.1 REV-2

CHANGE 1

(6-2.)

E.   Plan Coverage.  Plans are not required to warrant that

a covered property complies with (1) the original

dwelling plans and specifications; (2) applicable

building codes; or (3) specific terms of a homeowner's

contract to purchase a property.  However, the

protection plan must provide:

1)   During the first year of coverage, a warranty

against construction deficiencies in workmanship

and materials resulting from the failure of the

covered property to comply with standards of

quality as measured by acceptable trade practices,

as well as correct the problems with, or restore

the reliable function of, appliances and equipment

damaged during installation or improperly

installed by the builder.

"Construction deficiencies" are defects (not of a

structural nature) in a dwelling covered by an

insured ten-year protection plan that are

attributable to poor workmanship or to the use of

inferior materials which result in the impaired

functioning of the dwelling or some part thereof.

Defects resulting from homeowner abuse or from

normal wear and tear are not considered

construction deficiencies.

In instances of builder default, a Plan must

provide insurance coverage to resolve any builder

warranty obligation(s) that may affect the

structural components of the dwelling, including

site grading and drainage problems.

2)   During the first and second year of coverage, a

Plan must provide a warranty against defects in

the wiring, piping and ductwork in the electrical,

plumbing, heating, cooling, ventilating, and

mechanical systems.

3)   Basement slabs in "designated areas" must be

covered by a warranty in the Plan against damage

from the first through the fourth year.

The Secretary may designate any geographic area as

a "high risk area" where construction practices

6-10                 2/92

4145.1 REV-2

CHANGE 1

(6-2.E.3)

allow basement slabs to be placed on expansive or

collapsible soil.  By virtue of this authority,

the Secretary has designated the State of Colorado

as a "high risk area."

4)   From the first through the tenth year, a

structural defect must be covered by the Plan,

except for basement slabs in designated areas.

"Structural Defect" is defined as the actual

physical damage to the designated load-bearing

portions of a home caused by failure of such

load-bearing portions of the home that affects their

load-bearing functions to the extent that the home

becomes unsafe, unsanitary, or otherwise

unlivable.

Load-bearing components for the purpose of

defining structural defects are:

Footing and foundation systems; beams;

girders; lintels; columns; load-bearing walls

and partitions; roof framing systems; and

floor systems, including basement slabs in

homes constructed in designated areas

containing expansive or collapsible soils.

Beginning August 6, 1991, Plan Issuers are

required to cover roof sheathing on all FHA

insured properties .

Damage to the following non-load-bearing

components of the home is not considered a

structural defect:

Roofing; drywall and plaster; exterior

siding; brick, stone, or stucco veneer; floor

covering material; wall tile and other wall

coverings; nonload-bearing walls and

partitions; concrete floors in attached

garages; electrical, plumbing, heating,

cooling and ventilation systems; appliances,

fixtures and items of equipment; paint; doors

6-11                     2/92

4145.1 REV-2

CHANGE 1

(6-2.E.4))          and windows; trim, cabinets, hardware and

insulation.

Repair of a structural defect is limited to:

-    The repair of damage to designated

load-bearing portions of the home which is

necessary to restore their load-bearing

ability;

-    The repair of designated non-load-bearing

portions, items or systems of the home,

damaged by the structural defect, which make

the home unsafe, unsanitary or otherwise

unlivable (such as the repair of inoperable

windows, doors and the restoration of

functionality of damaged electrical,

plumbing, heating, cooling, and ventilating

systems); and

-    The repair and cosmetic correction of only

those surfaces, finishes and coverings,

original with the home, damaged by the

structural defect, or which require removal

and replacement attendant to repair of the

structural defect, or to repair other damage

directly attributable to the structural

defect.

It is the intent of this section to ensure

the repair of a covered home to a condition

approximating the condition just prior to the

defect, not to a like new condition.  It does

not require refinishing of all interior or

exterior surfaces if only one or two surfaces

are damaged.  It does not cover personal

property items, not a part of the structure,

which are damaged by the defect or as a

result of the defect.  It excludes damage

covered by a homeowner's casualty insurance

policy.

2/92                           6-12

4145.1 REV-2

CHANGE 1

(6-2.)

F.   Homeowner Complaints.  A Plan must assure timely

resolution of homeowners, complaints or claims.

Warranties set forth in a Plan must comply with Section

2301(a)(l)-(13) of the Magnuson-Moss Warranty--Federal

Trade Commission Improvement Act along with the

requirements and criteria set forth in this chapter.

1)   In the event of any dispute regarding a homeowner

complaint or structural defect claim, Plans must,

unless prohibited by applicable law, make

available binding arbitration proceedings arranged

through a nationally recognized dispute settlement

organization.  The Plan must also notify the

homeowner that they can request a review of the

disputed claim from the local HUD Field Office.

Under the Risk Retention Act, the domiciliary

State should respond to homeowner complaints

concerning insurance claims made to Plan Issuers.

If a State will not accept responsibility and

homeowner complaints are made to or become known

to the HUD Field Office, then the Field Office has

the authority to resolve such complaints.  In

reviewing these complaints, Field Offices should

contact the Plan Issuers for an explanation or

clarification of the complaint.  If the Field

Office is not satisfied with the Plan Issurer's

response, then report the findings to the Deputy

Assistant Secretary for Single Family Housing.

The sharing of arbitration charges will be as

determined by the Plan.  A Plan must make

available prearbitration conciliation procedures

at no cost to the homeowner, but arbitration,

which must be available to a homeowner during the

entire term of the coverage contract, must be an

assured recourse for a dissatisfied homeowner.

Plan Issuer must submit to the HUD Field Office

all arbitration requests for repairs in excess of

$10,000 or any arbitration request for structural

defects on FHA insured properties.  The HUD Field

Office will contact the Plan Issurer if additional

information is necessary about the nature of the

repair.  Arbitration awards in favor of the

builder or the Plan are subject to HUD review if

6-13                2/92

4145.1 REV-2

CHANGE 1

(6-2.F.1)

such review or appeal is requested by the

homeowner.  If HUD determines that the

arbitrator's decision is incorrect or inadequate,

then HUD can require the builder or the Plan to

correct the defect(s).  HUD's decision in this

matter is final.

2)   Exclusions from Plan coverage must not defeat

coverage objectives stated in this chapter and

must permit normal homeowner use of the covered

property, including normal maintenance and

emergency property protection measures.

G.   Deductible.  Unless prohibited by applicable law, Plans

must, at a minimum, stipulate that all homeowner

complaints covered by a Plan, including those regarding

construction deficiencies and structural defect claims,

will be settled in the amount of their actual cost to

correct or for the original sales price of the

property, whichever is the lesser.

1)   The claim may be subject to a deductible not to

exceed a total of $250.00 for all claims filed by

a homeowner during the first two years of coverage

and not to exceed a maximum of $250.00 per claim

during the third through the tenth year of

coverage.

A homeowner may be liable for a deductible if a

builder defaults on warranty performance and the

Plan Issuer has to make the covered corrections.

When the builder performs corrections under the

builder's warranty, no deductible included in the

Plan is applicable.  A claim review fee is not

allowed; therefore, a deductible cannot be paid by

the homeowner as an upfront claim review fee.  The

Plan can require the deductible after the extent

of the work is determined.

2)   In the case of claims filed by a condominium

association, a Plan must provide the condominium

association with an additional warranty that

allows for claims by homeowners involving the

common elements of the building.  The maximum

deductible for each claim is $250.00 per affected

2/92                          6-14

4145.1 REV-2

CHANGE 1

(6-2.G.2)

unit in the structure at the time of loss or

$5,000.00, whichever is less.

Recurrent claims for structural defects occasioned

by a common cause may be subject to a payment of

no more than one deductible.  The unit owner(s)

may file and pursue claim(s) involving individual

units either directly with the warrantor or

through a representative designated by the

condominium association.

Upon requesting approval of the Condominium

Project, the developer must show evidence of their

architects/engineers professional liability

insurance policy on all buildings over 13 units or

on all buildings designed using a construction

type other than the typical wood framing type

found in single family homes.

## 6-3. MAXIMUM LOAN-TO-VALUE RATIOS FOR DWELLINGS WITH APPROVED

WARRANTIES <TOP>.

This is acceptable for some homes less than a year old and

not approved for mortgage insurance or loan guarantee prior

to the beginning of construction.  This procedure does not

waive satisfaction of existing subdivision requirements.

A.   Application Processing.  Valuation Branch will process

each case as existing construction (commitment term = 6

months).  Refer to HUD Handbook 4115.3 REV-1, Master

Conditional Commitment - Master Appraisal Report

Procedure, for additional instructions.

1)   Dwellings that are not approved prior to beginning

of construction are not eligible for benefits

under Section 518(a) of the National Housing Act.

2)   The dwelling must satisfy the requirements that

would have been applicable if it had been approved

for mortgage insurance prior to the beginning of

construction.

6-15                2/92

4145.1 REV-2

CHANGE 1

(6-3.A.)

3)   Final inspection on properties "under

construction" or "existing less than one year

old":

a.   If the home is 100% complete:

The appraiser performs the appraisal and

completes the Uniform Residential Appraisal

Report (URAR) and all necessary exhibits.  In

this instance, the appraisal serves as the

final inspection and Form HUD 92051,

Compliance Inspection Report, is not

required.

Since the only additional work the appraiser

will do is to verify general conformance with

plans, an additional fee will not be allowed.

The appraiser will:

1.   Inspect for health and safety

violations.

2.   If there are no health and safety

problems and the property is ready for

occupancy, make the following statement

in the remarks section of the URAR:

"This is a newly completed dwelling

and appears to be in conformance

with the submitted construction

exhibits."

The appraiser is merely confirming that

if the plans call for a two story, three

bedroom, two bath house, that is what

was found on the lot.

3.   Take a color photograph of each

diagonally opposite front and rear

corner of the house to record adequate

grading and drainage of the site.  When

properly oriented, the photographs may

be the same as those used as appraisal

exhibits.  Make a statement on the URAR

2/92                          6-16

4145.1 REV-2

CHANGE 1

(6-3.A.3)a.3.)

of the acceptance of the grading and

drainage.

b.   If the property is under construction and not

100 percent complete:

The appraiser will perform the appraisal,

complete the URAR, require a final inspection

and make the following requirement as a

commitment condition:

"Property under construction; complete

according to submitted construction

exhibits."

In this case, a fee inspector will perform

the final inspection and furnish photographs

in accordance with paragraph 4-2.B.2)a. and

b.  Since these properties were not inspected

by HUD or VA prior to the start of

construction, the inspector makes the

following comment on Form HUD 92051:

"This is a newly completed dwelling that

was not completed under HUD or VA

inspections.  The dwelling appears to be

in conformance with the submitted

construction exhibits."

4)   The name of the warranty plan must be included

with the application for conditional commitment

for the inspection exception to apply.

5)   Builders Certification (Form HUD 92541) must be

provided (for both "under-construction" and

"existing" cases) along with one complete set of

plans and architectural exhibits (see paragraph

3-3).  Post-commitment and post-endorsement review

is performed by the Field Office according to

instructions in paragraph 3-3.K.4) and 3-3.K.5).

Exhibits are placed in the case binder.  See

paragraph 2-1.

6-17                  2/92

4145.1 REV-2

CHANGE 1

(6-3-A.)

6)   Builder/seller must provide a one-year warranty,

Form HUD 92544; include the one-year performance

guarantee on Form HUD 92544-A, in addition to the

warranty of the HUD accepted Insured Ten-Year

Protection Plan.

7)   A conditional commitment item must be added to

require the submission of evidence that an

acceptable ten-year protection plan is in effect

before insurance endorsement can occur.

B.   Post-Commitment/Post-Endorsement Review.  Architectural

staff reviews according to instructions in paragraph

3-3.  Also ensure the proper insurance binder from the

10-year insured protection plan is in the case binder.

C.   Firm Commitment Application.  Mortgagee includes its

certification that the borrower was notified in writing

that:

1)   The property was not approved for mortgage

insurance or loan guarantee prior to beginning of

construction; and

2)   HUD does not have statutory authority to provide

financial assistance for any future property

repairs.

D.   Coinsurance and Direct Endorsement Lenders.

1)   At application, the lender contacts the HUD Field

Office to determine whether the warranty is

acceptable.

2)   When submitting the loan for insurance

endorsement, provide copies of evidence that a HUD

accepted insured ten-year protection plan is in

effect.  Ensure that Builders Certification, Form

HUD 92541 (Appendix 2), and the Agreement to

Execute Builder's Warranty of Completion of

Construction, Form HUD 92541-A (Appendix 6), was

provided as described in paragraph 3-3.

Also include the mortgagee certification as

described in paragraph 6-3.C. above.              *

2/92                           6-18

Architectural Processing and Inspections for Home Mortgage Insurance

Directive Number: 4145.1

4145.1 REV-2

# CHAPTER 7 - ARCHITECTURAL DATA

7-1.  GENERAL <TOP>.  All information essential to processing by the

Architectural Branch is termed "architectural data".  The

Chief Architect is responsible for the preparation,

compilation, filing, indexing and maintenance of the data.

7-2.  ARCHITECTURAL REFERENCE DATA <TOP>.  Each field office should

ensure the availability of material to meet the requirements

for conditional commitment and complaint processing.  The

Office of Manufactured Housing and Construction Standards is

required to maintain all reference data listed in HUD

Handbooks and in the Code of Federal Regulations.  Consult

when necessary.

Each field office is required to maintain applicable editions

of:

Accepted Local Codes and Ordinances

Accepted State Codes

CABO One- and Two Family Dwelling Code

Electrical Code for One- and Two Family Dwellings,

NFPA 70A

Code of Federal Regulations - 24 CFR Part 200

HUD Minimum Property Standards

HUD Technical Publications

HUD Use of Materials Bulletins

HUD Materials Releases

HUD Technical Circulars

HUD Structural Engineering Bulletins

HUD Mechanical Engineering Bulletins

HUD Sanitary Engineering Bulletins

HUD Technical Study Reports

7-1                              3/90

4145.1 REV-2

7-3. INSPECTION GUIDE <TOP>.  Prepared by, or under the supervision of,

the Chief Architect.  It must be reviewed annually and

revised as necessary.  HUD employees of the Architectural

Branch, and fee inspectors are furnished a current copy that

contains the following information:

A.  General Instructions.

1)  Fee Inspector's duties.  Always be aware that the

public relations of HUD may be affected by a person's

attitude and conduct while serving in an official

capacity.

a.  Be courteous at all times during the performance

of the assignment;

b.  Be polite, pleasant and businesslike, showing no

partiality, avoiding argument and controversy;

c.  Suggest acceptable remedies to anticipated

non-compliances or construction errors; and

d.  Avoid making either favorable or unfavorable

comment regarding the work.

2)  Fee Inspector's responsibility is to determine that

construction quality of the physical property is

acceptable as security for an insured loan.

a.  Assure that the finished structure is

substantially the same as that upon which the

commitment is based; and

b.  Inspectors who accept construction showing

substandard workmanship, or non-compliance with

the accepted exhibits, are remiss in their

duties; and

c.  Mortgage risk may be increased as a result of an

inspector's negligence.

3)  Purpose of compliance inspections is to observe and

report whether the construction, or repairs,

substantially comply with the conditions of the

commitment and with sound construction practices.

3/90                            7-2

4145.1 REV-2

(7-3.)

B.   Completion Instructions for Form HUD 92051, Compliance

Inspection Report (see Handbook 4190.1).

C.   Inspection Checklist.  Provide a copy of the One and Two

Family Dwelling Inspection Guide.  Prepare an additional

inspection checklist to suit local conditions and include

items not contained in the One and Two Family Dwelling

Inspection Guide.  Briefly word each item to identify the

construction to be checked.  Do not describe action to be

taken.

D.   Checklist of Errors and Suggested Corrections.  Provide a

list of items frequently encountered during an

inspection.  Each non-compliance must give:

1)  The number of the statement to be checked on Part I

of the Compliance Inspection Report, Form HUD 92051.

2)  A brief description of the non-compliance, clearly

specifying the appropriate correction.

E.   List of Variations.  Provide items that the inspector

would normally report under Statement 10 - Part I of

Compliance Inspection Report.  Also provide a list of

minor variations that normally are not reported on the

Compliance Inspection Report.

F.   HUD Data.  Include any special technical data useful

during inspections.

G.   Field Office Memorandums.  Determinations on construction

techniques or materials can be included if considered

useful.

H.   Industry Data.  Issued by trade associations.  Do not

include material of a predominantly advertising

character.

7-3                             3/90

4145.1 REV-2

7-4.  MINIMUM PROPERTY STANDARDS (MPS) <TOP>.  These establish the

minimum quality acceptable to HUD, on a technical basis, in

all HUD Field Office jurisdictions.  No higher or lower

minimum level will be established by any field office without

Headquarters approval.  See HUD Handbook 4910.1, Minimum

Property Standards for Housing.

A.   Local Acceptable Standard (LAS).  Published by the field

office where an acceptable variation to the MPS is

considered necessary and is to be on a repetitive basis.

1)   The Field Office Manager recommends a LAS accepting

the variation by submitting a request to

Headquarters, Office of Manufactured Housing and

Regulatory Functions, Construction Standards Branch.

If the LAS appears justified and is acceptable, it

will be retyped with the necessary corrections and

forwarded to the field office for distribution.

2)   A LAS should be coordinated with the local Farmers

Home Administration and Department of Veterans

Affairs to eliminate varying interpretations.

It is also desirable to discuss any proposed LAS with

the local Home Builders Association.

3)   Occasionally, an application may be received that

will not meet HUD MPS or construction requirements.

Where compliance with a specific standard may be

impracticable or may cause unnecessary hardship:

a.  The field office may accept the variation without

the prior approval of Headquarters; and

b.  The variation must not be considered as

establishing a pattern or a precedent for similar

acceptance in repetitive situations.

B.  Other Applicable Standards.

1)  Handbook 4940.2, Minimum Design Standards for

Community Water Supply Systems,

2)  Handbook 4940.3, Minimum Design Standards for

Community Sewerage Systems.

3)  Handbook 4930.3, Permanent Foundations for

Manufactured Housing

APPENDIX 1 FILE IN PDF FORMAT <TOP>

APPENDIX 2 BUILDER’S CERTIFICATION <TOP>

APPENDIX 3 BUILDING CODE REVIEW <TOP>

APPENDIX 4 POST-ENDORSEMENT/POST-COMMITMENT REVIEW <TOP>

APPENDIX 5 AGREEMENT TO EXECUTE A BUILDER’S WARRANTY OF <TOP>

APPENDIX 6 EARLY START LETTER <TOP>

This is in response to your request for permission to start

construction on a proposed property, FHA case number ____________,

located at ______________________________________________________,

The Builders Certification (Form HUD 92541) and the Agreement

to Execute a Builder's Warranty of Completion of Construction

(Form HUD 92541- A), that you submitted, are acceptable.  You are

hereby advised that the property is approved for mortgage

insurance prior to the beginning of construction.  This

determination permits you, the builder, to begin construction

without affecting the amount of the maximum insurable mortgage.

This letter is not to be construed as a commitment for

insurance.  Final acceptance is always subject to site

acceptability.  Any commitments that may be issued will be

governed by the provisions of the National Housing Act and the

HUD Regulations in effect at the time of issuance.  The amount and

terms of the insurable mortgage will be governed by underwriting

processing.  The expiration date of any commitment will be _________

(9 or 12) months from the date of this letter.

Assuming that Form HUD 92800.5B, Conditional Commitment /

Direct Endorsement Statement of Appraised Value is issued, it will

set forth the value and the amount, terms and conditions as may be

prescribed by this office.  For the initial inspection, please

contact the following private fee inspector:

A complete copy of the construction exhibits must be on site,

and must be identical to those submitted to the fee appraiser for

valuation purposes.  Construction must not proceed beyond the

framing stage prior to the receipt Form HUD 92800.5B from this

office.  The framing stage is defined as a building that is

enclosed and roofed, structural framing completed and exposed, and

rough-in heating, plumbing and electrical work installed, and

visible for inspection.

Upon receipt of Form HUD 92800.5B, Conditional Commitment /

Direct Endorsement Statement of Appraised Value, please ensure

that all conditions are properly complied with.  Failure to

respond to all requirements could result in delays to the

endorsement of the loan and possibly affect HUD's insurable risk

on the property.

Your cooperation in regard to these requirements is

appreciated.

APPENDIX 7 ADDENDUM TO WARRANTY OF COMPLATION OF CONSTRUCTION <TOP>

APPENDIX 8 SITE GRADING AND DRAINAGE GUIDELINES <TOP>

Construction complaints and structural defect claims continue

to be numerous due to inadequate grading and drainage of the

sites.  The following guidelines, applicable to all newly

constructed properties, should be monitored by each field office

when performing required review of construction exhibits as they

relate to the builder certification program and compliance

inspections (both HUD processed and Direct Endorsement.  This

guideline should also be reviewed in training sessions with fee

and staff inspectors.

Building codes accepted under 24 CFR 200.926a do not cover

all site grading and drainage conditions.  However, 200.926d(b)(3)

Site Conditions, requires the property to be free of those

foreseeable hazards and adverse conditions which may affect the

health and safety of the occupants or the structural soundness of

improvements, or which may impair customary use and enjoyment of

the property.  200.926d(c)(2) and 200.926d(c)(4) include

additional site design requirements.

I.   OBJECTIVE.

A.   Drainage of Surface Water should be provided away from

all sides of all buildings and off the lot in a manner

that will:

1)  Minimize possibility of dampness in basements and

crawl spaces;

2)  Prevent adverse supporting soil behavior;

3)  Prevent soil erosion; and

4)  Prevent standing (or ponding) of water on site;

In arid areas, controlled ponding for irrigation is

permitted if emergency overflows are provided to

protect buildings.  Walks, driveways, retaining walls

and other improvements should be constructed so as

not to interfere with drainage.  Required walks

should not be used as drainage channels.

(I.)

B.   Site Grading and Drainage should:

1)   Provide suitable access from abutting street to the

dwelling and any accessory buildings;

2)   Provide immediate diversion of water away from

buildings and off the site;

3)   Avoid concentrating runoff onto neighboring

properties where erosion or other damage may be

caused;

4)   Provide usable outdoor space for occupants; and

5)   Minimize erosion.

II.   FINISH GRADING.

A.   General.  All unpaved lot areas, except those preserved

in an appropriate natural condition, should be fine

graded to provide smooth even surfaces conforming to

elevations noted below.  All visible rock and debris

should be removed prior to fine grading.

B.   Minimum Protective Slopes.  All walls and foundations of

buildings and any water-supply well should be provided

with protective slopes to assure immediate drainage and

diversion of surface water away from these structures and

off the site.

1)   Provide minimum fall of 6 inches away from structure

in 10 feet, except as restricted by side lot lines or

other major considerations, without regard to soil

type or ground frost conditions.  The horizontal

length of such slopes may be reduced as necessary at

building corners and side yards.

2)   Provide at least 6 inches in 25 feet (2% gradient)

in all other unpaved areas subject to ground frost,

expansive soils or collapsible soils, or at least 3

inches in 25 feet (1% gradient) in unpaved areas not

(II.B.))2

subject to such conditions.

If minimum slopes cannot be attained, paved

gutters or other drainage structures may be

installed as acceptable to the Director of

Housing (or designee) or DE Underwriter.

a.   All unpaved areas should slope continuously at

the above gradients to lower elevations off the

site, or to a drainage structure on the lot.

b.   Drainage swales or valleys formed by intersecting

slopes should have adequate depth, width and

longitudinal gradient to carry away the maximum

predictable volume of storm water runoff.

1.  Gradients of unpaved swales should not be

less than that required above for other

unpaved areas.

2.  Surface erosion protection should be in place

to prevent accumulation of water in critical

drainage swales.

c.   Where catch basins or inlets are installed, other

than at required basement areaways, finish grade

elevations of adjoining areas should provide for

emergency surface overflow so that, in event of

failure of catch basins or inlets, buildings and

window wells (or areaways) will be protected

against flooding.

d.   Areaways for basement windows, entrances, and

garage entrances should be provided with

effective drainage facilities.  Catchment area

should be as small as possible and must be

protected from overflow of stormwater from

adjacent areas.

e.   Roof drainage should discharge at least 5 feet

away from building walls when expansive,

collapsible or erodible soils are present.

(II.)

C.  Maximum Slopes.  Height and steepness of slopes and

maximum gradients of unpaved drainage channels should be

such as can be satisfactorily maintained without erosion

or land slippage and should provide satisfactory access

to and around the structure.

1)  For access around buildings and for maintenance of

building and lot improvements, provide an area

generally at least 4 feet wide with a gradient away

from building no steeper than 1 in 10 (10% gradient).

2)  Where considered necessary by Director of Housing (or

designee) or DE Underwriter, precautionary measures

should be taken (i.e., retaining walls, sodding or

planting, etc.) to stabilize the soil.

D.  Suitable Drainage Structures such as paved gutters, drain

inlets and subsurface drain lines should be installed

where necessary to protect against dampness, flooding,

erosion or other damage by surface water or ground water.

Drainage structures should be properly connected to

adequate outlets that are protected, where necessary, by

recorded permanent easements.

Perimeter foundation drains are necessary on all houses

with basements or potentially habitable living space

below finish exterior earth grade or in other situations

where water and/or soil conditions warrant their use.

Outlets must not permit backflow into subsurface drains.

III. WALKS, STEPS AND DRIVEWAYS.

A.  General.  A walk and any necessary step(s) should provide

safe and convenient use from house directly to the street

or to a driveway connected to a street.  Walk and step

construction should be of durable and appropriate

material, on stable, adequately drained subgrade or bed.

B.  Walk Design.

1)  Gradient should not be steeper than 1 in 20 (5%) in

areas subject to frequent freezing or 1 in 10 (10%)

in other areas.

(III.B.)

2)  Cross-slope should be adequate for immediate drainage

of surface water off the walk; required walk must not

be used as a drainage channel.

3)  Walk surfaces should be at or below adjacent ground

elevations.

C.   Step Design.

1)  Width should not be less than width of walk that is

served.

2)  A single step in a walk and any flight of steps of

more than 5 feet total rise should be avoided

wherever practical; substantial handrail of durable

construction must be provided if more than a 30-inch

rise in a single flight.

D.   Driveway should be provided from street or alley to the

garage or carport, if any; if no garage or carport, a

driveway will generally extend to rear line of dwelling.

In case of unusual difficulty or hardship, other parking

space acceptable to Director of Housing (or designee) or

DE Underwriter may be provided.

Construction should be with suitable subgrade, base,

drainage and surfacing so as to be durable under the use

and maintenance contemplated.

1)  Longitudinal gradient should not be less than one

percent nor steeper than permitted for walks if

driveway is also used as a required walk.  If used

only as a driveway, no less than one percent (1%) and

no steeper than 1 in 7 (14%), except that in areas of

heavy snow fall or ice build-up, longitudinal

gradient should not be steeper than 7%.  Crown, or

cross slope, should not be greater than 5%.

2)  Grade transitions should be provided at top and

bottom of a steep driveway to prevent dragging of

vehicle undercarriage or bumper guards.

3)  Gradient of a required parking space should not be

steeper than 1 in 20 (5%) nor less than one percent.

4145.1 REV-2

APPENDIX 9 CONSTRUCTION EXHIBIT SUBMITTAL <TOP>

APPENDIX 10 LIST OF HUD ACCEPTED <TOP>

APPENDIX 10                                                     CHANGE 1

___________________________________________________________________________

List of HUD Accepted

Insured Ten-Year Protection Plans

(As of November 25, 1991)

Name of Plan                               Insurance Backing

___________________________________________________________________________

BUILDER'S HOME WARRANTY, INC.             Robis Internationl Insurance

2131 Palamar Airport Road, Suite 300        Brokers, Inc. of

Carlsbad, CA 92009                          San Francisco, CA

(303) 843-0969

1-800-786-9130

Payment by builder is required

prior to or at closing.

___________________________________________________________________________

HOME BUYER'S WARRANTY CORPRATION          National Home Insurance Co.

(HBW 2/10) (HBW VI)                       (NHIC) of Colorado

1200 south Parker Road, Suite 200         (Risk Retention Group)

Denver, CO 80231

(303) 368-4805

1-800-942-1243

Use form HBW 202 for insurance

binder.  Payment by builder is

required at closing

___________________________________________________________________________

HOME OWNER'S WARRANTY CORPORATION         Home Owners Warranty Corp

(HOW)                                     (Risk Retention Group)

1110 N. Glebe Road, Suite 800

Arlington, VA 22201

(703)   516-4100

Use for HOW 503 for insurance binder.

Payment by builder not required if

form properly executed.

___________________________________________________________________________

MANUFACTURED HOUSING WARRANTY CORP.       ***MHWC in a sister company

(NM)                                    of Residential Warranty Corp.

P.O. Box 641                              and the insurance backer and

Harrisburg, PA 17108-0641                 state acceptance listed for

RWC are applicable for MHWC.

(717)   939-1971

1-800-247-1812

Use Final Enrollment form for

insurance binder, Payment by

builder is not required prior

to or at closing.

___________________________________________________________________________

i ( 1 of 4)                          2/92

___________________________________________________________________________

4145.1 REV-2

APPENDIX 10                                                     CHANGE 1

___________________________________________________________________________

List of HUD Accepted

Insured Ten-Year Protection Plans

(As of November 25, 1991)

Name of Plan                               Insurance Backing

___________________________________________________________________________

MID-ATLANTIC INSURANCE CORP.              Colony Insurance Company

(HW 10)

2117 Smith Avenue

Chesapeake, VA 23320

(804) 424-6523

Use form HW 114 for insurance

binder.  Payment by builder

not required if form properly

executed.

___________________________________________________________________________

NEW JERSEY NEW HOME WARRANTY PLAN         State of New Jersey

State of New Jersey

Department of Community Affairs

Division of Codes and Standards

Bureau of Homeowner Protection

CN 805

Trenton, NJ 08625-0805

(609)   530-6357

Payment by builder is required

prior to or at closing

___________________________________________________________________________

___________________________________________________________________________

i ( 2 of 4)                          2/92

___________________________________________________________________________

4145.1 REV-2

APPENDIX 10                                                     CHANGE 1

___________________________________________________________________________

List of HUD Accepted

Insured Ten-Year Protection Plans

(As of November 25, 1991)

Name of Plan                              State Acceptance

___________________________________________________________________________

BUILDER'S HOME WARRANTY, INC.             *California

2131 Palamar Airport Road, Suite 300

Carlsbad, CA 92009

(*Acceptable for Condominiums

(303) 843-0969                                and manufactured (mobile)

1-800-786-9130                                homes)

Payment by builder is required

prior to or at closing.

___________________________________________________________________________

HOME BUYER'S WARRANTY CORPRATION          *District of Columbia and all

(HBW 2/10) (HBW VI)                     states except Alaska and Hawaii

1200 south Parker Road, Suite 200

Denver, CO 80231

(303) 368-4805                            (*Acceptable for condominiums and

1-800-942-1243                                manufactured (mobile) homes)

Use form HBW 202 for insurance

binder.  Payment by builder is

required at closing

___________________________________________________________________________

HOME OWNER'S WARRANTY CORPORATION         *All states, the District of

(HOW)                                     Columbia and all Territories

1110 N. Glebe Road, Suite 800

Arlington, VA 22201

(703)   516-4100                          *Acceptable for Condominiums)

Use for HOW 503 for insurance binder.

Payment by builder not required if

form properly executed.

___________________________________________________________________________

MANUFACTURED HOUSING WARRANTY CORP.       ***MHWC in a sister company

(NM)                                    of Residential Warranty Corp.

P.O. Box 641                              and the insurance backer and

Harrisburg, PA 17108-0641                 state acceptance listed for

RWC are applicable for MHWC.

(717)   939-1971                          (Not acceptable for condominiums)

1-800-247-1812

Use Final Enrollment form for

insurance binder, Payment by

builder is not required prior

to or at closing.

___________________________________________________________________________

i ( 3 of 4)                          2/92

_____________________________________________________________________

4145.1 REV-2

APPENDIX 10                                                     CHANGE 1

___________________________________________________________________________

List of HUD Accepted

Insured Ten-Year Protection Plans

(As of November 25, 1991)

Name of Plan                               State Acceptance

___________________________________________________________________________

MID-ATLANTIC INSURANCE CORP.               Virginia*

(HW 10)

2117 Smith Avenue

Chesapeake, VA 23320

(804) 424-6523                             (*Acceptable for condominiums)

Use form HW 114 for insurance

binder.  Payment by builder

not required if form properly

executed.

___________________________________________________________________________

NEW JERSEY NEW HOME WARRANTY PLAN          New Jersey

State of New Jersey

Department of Community Affairs

Division of Codes and Standards

Bureau of Homeowner Protection             (*Acceptable for condominiums)

CN 805

Trenton, NJ 08625-0805

(609)   530-6357

Payment by builder is required

prior to or at closing

___________________________________________________________________________

___________________________________________________________________________

i ( 4 of 4)                          2/92

_____________________________________________________________________

4145.1 REV-2

CHANGE 1                                               APPENDIX 10

___________________________________________________________________________

List of HUD Accepted

Insured Ten-Year Protection Plans

(As of November 25, 1991)

Name of Plan                              Insurance Backing

___________________________________________________________________________

PREFERRED BUILDERS WARRANTY CORP.         Sphere Drake PLC

(PBW)                                     Insurance Company

4700 N.W. 2nd Avenue, Suits 202

Boca Raton, FL 33431

(407) 994-2017

Payment by builder in required

prior to or at-closing.

___________________________________________________________________________

QUALITY BUILDERS WARRANTY CORP            Merchants and Business Men's

(QBW)                                   Insurance Company of

P.O. Box 271                                Harrisburg, PA

Harrisburg, PA 17011

(717) 737-2522

1-800-334-9143

Confirmation of Enrollment form

for insurance binder.  Payment

by builder is not required if

form is properly executed.                                                (

___________________________________________________________________________

RESIDENTIAL WARRANTY CORPORATION          Western Pacific Mutual

(RWC)                                     Insurance Company of

P.O. Box 641                                Denver, Colorado

Harrisburg, PA 17108-0641                   (Risk Retention Group)

(717) 939-1971

1-800-247-1812

Use Final Enrollment form                 Manufactured Housing Warranty

for insurance binder.                     Corp. (MHWC) is a sister

Payment by builder is not                 company of RWC and the same

required prior to or at                   states and insurance backers

closing                                   are applicable.

___________________________________________________________________________

WARRANTY UNDERWRITERS INSURANCE           Self

COMPANY (HOME)

6161 Savoy Drive, Suite 820

P.O. Box 741808

Houston, TX 77274-1808

(713) 785-1314

***************************************************************************

NOTE:  The above ten-year insured warranty companies have agreed to

comply with the provisions of the new regulations in 24 CFR

Part 200 and the insured ten-year protection plan requirements

outlined in HUD Handbook 4145.1 REV-2, Chapter 6

___________________________________________________________________________

ii ( 1 of 2)                           2/92

_____________________________________________________________________

4145.1 REV-2

CHANGE 1                                               APPENDIX 10

___________________________________________________________________________

List of HUD Accepted

Insured Ten-Year Protection Plans

(As of November 25, 1991)

Name of Plan                               State Acceptance

___________________________________________________________________________

PREFERRED BUILDERS WARRANTY CORP.          Florida

(PBW)

4700 N.W. 2nd Avenue, Suits 202

Boca Raton, FL 33431                       (Acceptable for condominiums)

(407) 994-2017

Payment by builder in required

prior to or at-closing.

___________________________________________________________________________

QUALITY BUILDERS WARRANTY CORP             Connecticut, Delaware, Georgia,

(QBW)                                    Kentucky, Maryland,Massachusetts

P.O. Box 271                               New Jersey, Now York, North

Harrisburg, PA 17011                       Carolina, Ohio, Pennsylvania,

South Carolina, Tennessee,

(717) 737-2522                             Virginia and Wast Virginia

1-800-334-9143

Confirmation of Enrollment form

for insurance binder.  Payment

by builder is not required if             (All states are acceptable for

form is properly executed.                          condominiums)

___________________________________________________________________________

RESIDENTIAL WARRANTY CORPORATION           District of Columbia and all

(RWC)                                    states except Colorado.

P.O. Box 641

Harrisburg, PA 17108-0641                  (Acceptable for condominiums)

(717) 939-1971

1-800-247-1812

Use Final Enrollment form                  Manufactured Housing Warranty

for insurance binder.                      Corp. (MHWC) is a sister

Payment by builder is not                  company of RWC and the same

required prior to or at                    states and insurance backers

closing                                    are applicable.

___________________________________________________________________________

WARRANTY UNDERWRITERS INSURANCE            Texas

COMPANY (HOME)

6161 Savoy Drive, Suite 820

P.O. Box 741808

Houston, TX 77274-1808

(713) 785-1314

***************************************************************************

NOTE:  The above ten-year insured warranty companies have agreed to

comply with the provisions of the new regulations in 24 CFR

Part 200 and the insured ten-year protection plan requirements

outlined in HUD Handbook 4145.1 REV-2, Chapter 6

___________________________________________________________________________

ii ( 2 of 2)                           2/92

*U.S. Government Printing Office: 1992 - 312-218/60073

# APPENDIX 11 ARCHITECTURAL REQUIREMENTS GRID

<TOP>

APPENDIX 11

___________________________________________________________________________

Architectural Requirements Grid

__________________________________________________________________________

Column         A     B     C     D     E     F      G       H        I

__________________________________________________________________________

Construction Plans Subdiv Insp Final  Bldr  Bldr 10-Year Termite   Carpet

Status       Reqd   Req   Reqd Insp   Cert  Warr  Warr   Control I.D., Mfg

only                              Warr &

Ins Certs

__________________________________________________________________________

PROPOSED CONSTRUCTION

High Ratio    (7)   (6)   (3)   (3)    (9)  (10)   (3)     (5)      (11)

(1)

__________________________________________________________________________

UNDER-CONSTRUCTION

High Ratio    (7)   (6)   (2)  (2)(4)  (9)  (10)   (4)     (5)      (11)

Low Ratio     (8a)  (6)   N/A  (2)(8a) N/A   N/A   N/A     (5)       N/A

__________________________________________________________________________

EXISTING CONSTRUCTION LESS THAN ONE YEAR OLD (13)

High Ratio    (7)   (6)   (2)   (4)    (9)  (10)   (4)     (5)      (11)

Low Ratio     N/A   (6)   N/A   (8b)   N/A  N/A    N/A     (5)       N/A

__________________________________________________________________________

REQUIREMENTS for MANUFACTURED (Mobile) HOME for Title II - See Specific

Notes, Item 12

__________________________________________________________________________

Abbreviations:  Insp - Inspections;  Bldr - Builder; Cert - Certification;

Warr - Warranty; Mfg - Manufacturer's;

I.D. - Identification Number

__________________________________________________________________________

GENERAL NOTES

Use this Grid in conjunction with HUD Handbook 4145.1 .

Lenders can be assigned case numbers even though a builder is not on the

list of certified builders.  However, after the appraisal, the

construction exhibits must be reviewed by the HUD Field Office to

determine acceptance of the Builder's Certification prior to the builder

being placed on the list. (Refer to paragraph 3-3).

___________________________________________________________________________

i                              3/90

___________________________________________________________________________

4145.1 REV-2

APPENDIX 11

___________________________________________________________________________

Conversions from VA-CRV's (or VA-MCRV's).  Builders do not have to be on

the list of certified builders.  However, when submitting for mortgage

insurance, the following forms must be present:  Form HUD 92541, Form HUD

92541-A, Form HUD 92544 and Form HUD 92544-A.  (Refer to Paragraph

3-3.D.).

( ).  See Specific Notes listed on the following page.

SPECIFIC NOTES

1.  All cases approved prior to the beginning of construction or issuance

of an Early Start letter by HUD or VA are eligible for "high

loan-to-value" financing.  Section 518(a) structural defect warranty

applies.  (Refer to paragraph 6-1).

_________________________________________________________________________

___________________________________________________________________________

i (Continued)                      3/90

_____________________________________________________________________

4145.1 REV-2

APPENDIX 11

___________________________________________________________________________

2.  If the appraiser has no listed conditions requiring an inspection

(i.e., the property improvements are 100 percent complete), then the

appraisal (URAR) serves as the final inspection.

If the property is not 100 percent, a final inspection by a fee

inspector is required.  (Refer to paragraph 6-3).  All 3 inspections

may be required depending on the dwellings stage of construction.

3.  Three (3) inspections are required.  However, if the local

jurisdiction has been approved by the HUD Field Office, or if a HUD

accepted insured 10-year protection plan is provided, only a final

inspection may be permitted.

The 10-year protection plan may be used to request a waiver of the

initial and intermediate inspections.  A 10-year protection plan is not

required to obtain "high loan-to-value" financing on a "proposed

construction" case.  Section 518(a) applies whether or not a 10-year

protection plan is provided.  (Refer to paragraphs 4-3 and 4-4).

4.  A HUD accepted 10-year protection plan is required in order to obtain

"high loan-to-value" financing.  Section 518(a) of the National

Housing Act does not apply.  Providing a HUD accepted insured 10-year

protection plan does not change this provision.

Lender must notify borrower that the property was not approved by HUD

(or VA) prior to the start of construction and the Department does not

have authority to provide financial relief for any future property

repairs.  (Refer to paragraph 6-1).

5.  In areas of the country where termites exist, it is necessary to

control termite infestation.  Consult with local HUD Field Office for

requirement(s).

If termite soil treatment is used to control infestation, submit a

builders five (5) year guarantee on Form HUD 92052.  (Refer to Form HUD

92800.5B, item 16 of the Specific Commitment Conditions).

6.  Subdivision requirements must be followed (See Handbook 4135.1).

7.  Construction Exhibits must be submitted in the HUD Case Binder for all

properties receiving "high loan-to-value" (97/95%) financing.  (Refer

to Chapter 2).

On VA-CRV's (VA-MCRV's), plans are not required.  VA is responsible for

complaint processing during the one-year warranty period.  (Refer to

paragraph 3-3.D.).

___________________________________________________________________________

3/90                              ii

_____________________________________________________________________

4145.1 REV-2

APPENDIX 11

___________________________________________________________________________

8.  Where "low loan-to-value" (90%) financing is applicable:

a.  Under-Construction Case.  Final inspection by a fee inspector is

required.  Plans are required to ensure compliance with appraisal

conditions.  Certifications, warranties or guarantees are not

required by HUD.

b.  Existing Property (Less than one-year old).  Since the property

improvements are 100% complete, the appraisal (URAR) serves as the

final inspection.  Plans, certifications, warranties or guarantees

are not required by HUD.

9.  Submit Builder's Certification, Form HUD 92541, and the Agreement to

Execute the Warranty of Completion of Construction, Form HUD 92541-A.

(Refer to paragraph 3-3).

10. Submit the Warranty of Completion of Construction, Form HUD 92544, and

the Addendum to Warranty of Completion of Construction, Form HUD

92544-A. (refer to paragraph 3-18).

11. Submit Carpet Identification Number for compliance with Use of

Material Bulletin 44d.  Also provide the homebuyer all manufacturers

warranties and a certificate from the company that installed the

insulation, stating the R-values of the insulation used.  (Refer to

Form HUD 92800.5B, items 5, 10, and 14 of Specific Commitment

Conditions).

___________________________________________________________________________

3/90                               ii (Continued)

_____________________________________________________________________

4145.1 REV-2

APPENDIX 11

___________________________________________________________________________

12.  Manufactured (Mobile) Homes, Title II (Refer to paragraph 3-4)

a.  FHA Mortgage Insurance can only be obtained on manufactured homes

installed on a permanent foundation, built after June 15, 1976,

in conformance with the Federal Manufacturers Home Construction

and Safety Standards (FMHCSS).

b.  Comply with the Architectural Requirements, Specific Notes, for

the appropriate Construction Status, except the following:

1)  Column C; only two (2) inspections are required, an initial

inspection of the permanent foundation and a final inspection

of the entire property.

2)  Column I; only the manufacturers warranties are required.

At final inspection, the manufactured unit will be inspected to

ensure completion of on-site work (i.e., proper joint connections

and finish work associated with the joint connections).  If a

structural defect in the unit is observed, the final inspection

will not occur until the applicable State Administrative Agency

has verified structural soundness of the manufactured unit.

c.  Submit all information required in HUD Handbook 4930.3, Permanent

Foundations Guide for Manufactured Housing.  For an "existing"

property, the foundation design information in Appendices A, B

and C, may be used to verify the design of the existing system to

ensure compliance with the definition of a "Permanent

Foundation."

d.  The manufactured unit must not have been installed or occupied

previously at any other site or location.

1)  Existing Unit with New Permanent Foundation Proposed.

"High loan-to-value" Financing is allowed provided the above

mentioned items are submitted and accepted prior to issuance

of Form HUD 92800.5B, Conditional Commitment / Direct

Endorsement Statement of Appraised Value.  Comply with item

12.C.  Builder's Certification and Warranties are required

on the foundation.

2)  Existing Unit with Permanent Foundation Under-Construction or

Completed Less than One Year (see note 13).

Limited to "low loan-to-value" financing unless a HUD

accepted insured 10-year protection plan is provided (See

Appendix 10).  Comply with item 12.C above.  Builder's

Certification and Warranties are required on foundation if

"high loan-to-value" financing is provided with an insured

10-year protection plan.

___________________________________________________________________________

iii                           3/90

___________________________________________________________________________

4145.1 REV-2

APPENDIX 11

___________________________________________________________________________

3)  Existing Unit with a Permanent Foundation over One-Year Old

Comply with item 12.C above.  Builder's Certifications and

Warranties are not required.  "High loan-to-value" financing

is allowed because the manufactured unit has been on the

foundation for over one year.

13.  The beginning date for the term "less than one year" is the earliest

of:

a.  Date of Certification of Occupancy, or

b.  Date of completion on building inspection card, or

c.  Date of letter from local (or state) authority showing acceptable

completion of construction.

If the above dates are not available, provide sufficient

documentation establishing final completion of construction.

__________________________________________________________________________

iii (Continued)                    3/90